# Air Intake and Exhaust Design for Formula SAE Car

Needs Assessment, Concept Generation and Selection and Specifications

Senior Design

October 4, 2001

James Cameron

Jason Esquivel

Anders Gerhart

Don Morton

- Needs Assessment

1.1  Need Statement

Design of an intake and exhaust system for a prototype racecar, scheduled for mass production at minimal cost for the weekend autocross racer.  Intake and exhaust system must provide maximum horsepower and torque across the power band, provided a restricted intake and decibel limits placed on the design.

  - Demands

|  | Specification | Score |
| --- | --- | --- |
| Need | Air restrictor on intake | 10 |
| Need | Restrictor before engine and after throttle body | 10 |
| Need | Exhaust not to scrape ground | 10 |
| Need | Exhaust outlet to be behind centerline rear axel | 10 |
| Need | Exhaust not too high | 10 |
| Need | Exhaust not too loud | 10 |
| Need | Driver not subjected to exhaust fumes | 10 |

1.3  Wishes

|  | Specification | Score |
| --- | --- | --- |
| Want | Maximize air intake into engine | 8 |
| Want | Free flow air filter | 5 |
| Want | Good fuel atomization | 9 |
| Want | Runner volume equal to cylinder displacement | 9 |
| Want | Insulated exhaust | 7 |
| Want | Maximize flow of exhaust | 7 |
| Want | Minimum back pressure in exhaust | 7 |
| Want | Minimal exhaust bends | 7 |
| Want | Material to be composites and/or light metals | 6 |
| Want | Least restricting muffler | 6 |
| Want | Cool air intake | 5 |
| Want | Maintain laminar air flow to injector | 8 |

- Concept Generation and Selection

2.1  Intake Manifold

2.2  Exhaust Manifold

3.0 Specifications

| Diameter of restrictor | 20 mm |
| --- | --- |
| Exhaust ground clearance | 60 mm (min) |
| Exhaust outlet height above ground | 80 cm (max) |
| Exhaust outlet distance behind rear axel centerline | 60 cm (max) |
| Exhaust sound level | 110 db (max) |
| Runner volume | 150 cc |
| Number of intake ports | 4 |
| Number of exhaust ports | 4 |
| Volumetric efficiency (delivered to engine) | 75% +/- 5% |
| Number of fuel injectors | 4 |
| Exhaust outlets | 1 |