### Tools required

- Flat head screw driver

- 7mm Wrench

# Package Inspection

Verify that the parts were not damaged during delivery by carefully unpacking all items in the shipping carton and look for obvious signs of damage.  If any components are damaged repack parts in original package and contact your local distributor.

# WARNING !

Burn Hazard - Do not do any work on engine if it is hot.

Allow sufficient time for engine to cool after

car has been run.

Parts inspection

## Intake

# Plenum

- Fasten four (4) hose clamps to engine intake ports.

- Insert runner ends directly into hose clamps, as seen in figure.

  - Be sure that intake curves towards top of car, not towards back of car.

- Tighten down hose clamp screws.

  - Be careful not to over tighten screws, this can damage the hose clamps.

- Screw intake pressure sensor into plenum, as seen in figure.

- Make electrical connections for pressure sensor.

Fuel Injectors

- Apply a small amount of oil to all O-rings.

- Insert the fuel injectors into the fuel rail.

  - Be careful not to position the fuel injectors backwards, the electrical contacts should point towards the fuel rail.

- Insert free end of injectors into injector mounts on the runners of the plenum.

- Tighten fuel rail to engine block using tightening bolts.

# Throttle body

- Verify that all parts move, as seen in picture.

  - Fully opens

  - Fully closes

  - Moves smoothly through range of motion

- Check throttle cable for damage.

- Slide hose clamp 1 over restrictor intake to midpoint of hose clamp.

- Tighten hose clamp screw around restrictor.

  - Be careful not to over tighten hose clamp screw.

- Insert small end of throttle body into hose clamp until it touches the restrictor intake, inside the hose clamp.

- Tighten the hose clamp screw around throttle body.

- Fasten throttle cable to throttle.

- Connect throttle position sensor (TPS) to throttle body using two (2) 1/8” bolts.

- Connect electrical harness of  TPS to connecting plug.

# Air filter

- Slide air filter over large end of throttle body.

- Tighten hose clamp screw around throttle body.

  - Be careful not to over tighten hose clamp screw.

## Exhaust

# Header pipes

- Slide mounting plates up to collars on ends of exhaust headers.

- Align bolt holes in mounting plates to bolts on cylinder head.

- Insert exhaust collars into exhaust ports on cylinder head

- Tighten all nuts firmly against mounting plates using 7mm wrench.

  - Tighten nuts to 12 N*m

# O2 sensor

- Screw O2 sensor into exhaust pipe.

- Make electrical connections.

# Muffler

- Slide muffler onto end of exhaust pipe.

- Use clamp 3 to connect muffler to header.

### Warning !

Make sure Fuel pump is not on.

Battery key is in disable position.

Do not test until the exhaust has been installed.

Engine must be cool.

Before stating engine, leave ignition off and turn on the fuel pump. Once full pressure is obtained check for fuel leaks. After checking, the engine is ready to start. Start engine and let idle Confirm no leaks between the runner and fuel rail and exhaust is exiting out the muffler.