## */

## Laboratory 5: Gear Formulae

## Group Members:

,

- Verify the table below. Add the equations for KV, and any other terms you think are relevant. Then generate similar tables for the contact stress, allowable bending stress, and allowable contact stress. One person is to generate a table, the other is to check it. Then swap for the next, and so on. Note that these sheets might be helpful for the first test. Note that the tables and graphs will be provided for the test – you only need the table or figure reference number.

## Bending Stress (US)

σb=WtKaPdKSKmKBKIKvFJ

| Term | Description | Formula/location/figure |
| --- | --- | --- |
| V | Pitch line velocity | πdω12, d in in, ω in rpm |
| Wt | Transmitted load | 33000 HV, H in hp, V in ft/min |
| Ka | Application factor | Table 11-17, page 721, |
| Pd | Diametral Pitch | Nd |
| KS | Size Factor | Usually 1 unless otherwise stated, page 720 |
| Km | Load distribution factor | Table 11-16, page 720, |
| KB | Rim thickness factor | Equations 11.20a,b, page 721, mB=tRhtKB=−2mB+3.4 0.5≤mB≤1.21 mB>1.2 |
| KI | Idler factor | Page 721, KI=1 nonidler1.42 idler |
| KV | Dynamic factor |  |
| F | Face width | usually given, recommended values: |
| J | Geometry factor | Tables 11-8 through 11-15 |

Generated by:

Checked by:

## Bending Stress (US)

## Contact Stress

Generated by:

Checked by:

## Allowable Bending Stress

Generated by:

Checked by:

## Allowable Contact Stress

Generated by:

Checked by: