FAMU-FSU College of Engineering

Society of Automotive Engineers Student Chapter

Senior Design Team #2

SAE II: Cockpit Ergonomics

Design Specifications

October 2, 2003

Advisor:

Dr. Patrick Hollis

Team:

Patrick Middleton

Christopher Hokes

The following specifications were determined by the needs put forth by the customer and to accommodate the SAE rules for both competitions.

General

- Adjustable to Driver

  - Smallest driver 5’ 5”

  - Largest driver 6’ 3”

- Allow for a quick egress

  - Max 5 seconds

- Driver must be securely restrained

- Driver must be able to reach all controls when buckled in

- Must be sufficiently comfortable to prevent driver fatigue during the race

- The steering wheel and shifter should be clear of the driver at all times

- All gauges should be clearly visible to the driver

Testing will be done to determine the needed steering wheel, shifter, pedal assembly, and seat position.

Mini Baja

- Firewall (32.3)

  - Must be between cockpit and the engine and fuel tank

  - Must be metal, at least 0.508 mm (0.020 in.) thick

- Belly Pan (32.4)

  - Must cover entire length of cockpit

  - May be metal or equivalent plastic sheet

- Leg and foot shielding (32.5)

  - All steering and suspension links must be shielded from driver’s legs

  - Driver’s feet must be completely in the roll cage

- Kill switches (32.6)

  - Each vehicle must have two easily accessible kill switches effecting the ignition and the entire electrical system

  - Must have a cockpit and external kill switch

  - Must be labeled as “kill switch”

- Fire extinguisher (32.7)

  - Must have a minimum UL rating of 5 B-C

  - Must be mounted in cockpit below driver’s head

  - Mountings must be designed to resist shaking loose over rough terrain

  - Must be equipped with a manufacturer installed dial pressure/charge gauge

  - Must be labeled with school name and car number

- Throttle (32.8)

  - Only foot operated throttle controls are allowed

  - Push/pull style cable

  - Must be positioned to avoid foot entrapment

  - No driver may add extensions to the control surface or the driver

- Restraints (33.1)

  - Four strap restraint system is required

  - Must have arm restraints

  - Belts cannot be more than three years old

  - All belts must be joined with a single metal-to-metal quick release lever type buckle

- Brakes (34)

  - Single foot brake

  - Two independent hydraulic systems in case of failure

- Driver equipment (39)

  - Driver must wear a helmet with Snell M-2000 or SA-2000 rating, neck support, and goggles

  - Must wear appropriate clothing, including long pants, socks, shoes, gloves, and a long sleeved upper garment

  - For deep water events, each driver must wear a U.S. Coast Guard approved life jacket

Formula (2004 FSAE rule)

- Brakes (3.2.5)

  - Operated by a single control

  - Must have two independent hydraulic circuits

  - Must have an analog brake over travel switch that is capable of killing the ignition and cutting power to any electrical fuel pumps

- Throttle (3.5.4.2)

  - Must be mechanically actuated (e.g. via a cable or a rod system)

  - Must have at least two return springs

  - Incorporate a positive pedal stop

  - Have a pedal capable of forcing the throttle closed (e.g. toe strap)

- Driver restraint (3.4.1)

  - Use either a five or six point restraint system, meeting SFI specification 16.1

  - Must be mounted according to FSAE rule 3.4.1 (D,E,F)

  - Must include arm restraints

- Safety equipment (3.4.2)

  - A closed face helmet integrating a full face shield or goggles with a Snell SA-2000 or M-2000 rating or better must be worn

  - A fire resistant suit with gloves and shoes meeting SFI 3.2A/1 standards or higher

- Driver visibility (3.4.3)

  - Minimum of 200 degrees field of vision by either driver turning his head or the use of mirrors

- Head restraint (3.4.4.1)

  - Minimum of 36 square inches

  - Minimum 1.5 inches of padding

  - Maximum of 1 inch away from drivers helmet

  - Able to withstand an 890 N or 200 lb force applied in the rearward direction

- Role bar padding (3.4.4.2)

  - Any portion of roll bar that might be able to contact the drivers helmet must be padded with material at least 0.5 inches thick

- Floor closeout (3.4.5)

  - Must completely separate driver from pavement

  - Maximum gaps between panels 1/8 inch

- Steering wheel (3.4.6)

  - Near circular perimeter

  - Must use a quick disconnect

- Driver egress (3.4.7)

  - Max 5 seconds

- Master switch (3.4.9)

  - Must be in easy reach of driver

  - Properly labeled as per FSAE rule 3.4.9.3

- Firewall (3.4.10)

  - Must completely separate driver compartment from all components of: fuel supply, engine oil, and coolant system

- Accessibility of controls (3.4.12)

  - All vehicle controls must be accessible from inside the cockpit without any part of the driver being outside any major structural members of the vehicle