Filtration System Frame

Materials:					Qty:

1 X ½  X 6  White Wood 		                     	   2

1 X 2 X 8  White Wood 		                     	   2

2 X 4 X 8  White Wood				   2

2 X 6 X 8  White Wood				   1

2 X 4 X 10 White Wood				   1

¼ X 4 X 8 Drywall				   2

8 X 2 ½  Drywall screws				   1 box

Corrugated Strips				   1 box

Tools/ Equipment:

Drywall Saw

Power Drill

Hammer

Procedure:

- Cut 2 X 4 X 10 into 3, 33 inch sections

- Cut both sheets of drywall into 3 feet sections

- Dimension the intake and exhaust vent positions onto the drywall

- Cut out the vent section in the drywall

- Cut the 1 X ½  X 6 boards into the following rectangle inner dimensions:  1 X 2 feet  and a 1 X ½ feet

  - Cut a 45 degree angle at all ends of boards; also, ensure inner dimensions are correct

- Insert corrugated strips into the previously cut pieces of wood (vent support )

- Screw the 2 X 4 X 8 boards to the three 33 inch sections forming a rectangular frame

  - Place the boards at each end with the third in the middle

- Screw the first sheet of  drywall onto frame

- Screw both vent supports onto the drywall

- Prime Drywall and dimension section to be painted

- Paint Drywall and let dry

- Cut 1 X 2 X 8  into two 3 feet sections (support bracket)

  - The 45 degree angle must face opposite sides

- Cut 2 X 6 X 8 into half’s, prime and paint

- Mount Frame on top of  the two 2 X 6 X 4

- Screw down frame onto boards

- Screw Support bracket at the 2 X 6 X 4 board and at the frame; on both sides

- Screw second sheet of drywall onto the back side of the frame