3rd Deliverable

Group # 16

Rabei Rabei

Michael Boyle

Michael LaBeach

Andrew Magaletti

October 2, 2008

Table of Contents

Project Specifications……………………………………………………….page 3

Component Tables……………………………………………………………page 4

Controllers ………………………….…………………....page 4

Motors……………………………………………….……...page 5

Batteries…………………………………....................page 5

Cost Chart…………………………………………………..page 6

Options……………………………………………………....page 7

References……………………….……………………………………..….....page 8

Project Specifications

- Customer needs

  - The newly converted car must range 50 miles.

  - Achieve a speed of approximately 75mph.

  - Travel up a 15° incline for a minimum of 30secs at 20mph.

- Create a gas to electric driven automobile manual

  - Section on the conversion process

    - Procedures taken to achieve the conversion process

  - Section on troubleshooting problems that may arise

    - A list of possible problems

    - Helpful hints to troubleshoot

- Incorporate alternative green energy into the car

  - Focusing primarily on wind and solar energy

- Rewiring

  - Headlights and main power lines

    - Reconfigure headlight system

    - Relocating the extra batteries to the front of the vehicle

      - Routing wire to original battery pack

Controllers

| Model | Minimum Voltage | Maximum Voltage | Maximum Current | Water/Air Cooled | Cost | Vendor |
| --- | --- | --- | --- | --- | --- | --- |
| Kelly KDH09500A | 24V | 96V | 500A | Air | $799.00 | http://www.kellycontroller.com/mot/High-Voltage-DC-Motor-Controller.html |
| Kelly KDH12500A | 24V | 120V | 500A | Air | $899.00 | http://www.kellycontroller.com/mot/High-Voltage-DC-Motor-Controller.html |
| Kelly KDH14600B | 24 | 120V | 600A | Air | $1145.00 | http://www.kellycontroller.com/mot/High-Voltage-DC-Motor-Controller_Iso.html |
| Curtis 1231C-7401 | 72V | 120V | 400A | Air | $1050.00 | http://www.evparts.com |
| Curtis 1231C-7701 | 72V | 120V | 550A | Air | $1397.25 | http://www.evparts.com |
| LogiSystems | 72V | 120V | 550A | Air | $1209.88 | http://www.evsource.com/tls_logisystems72-120V.php |

The minimum controllers needed to upgrade the car to handle the 96Vs that are going to be required to get the car to the appropriate speed. The Alltrax controller currently in the car can handle a max voltage of 72V. The table above shows some controllers that will be able to handle a 96V battery pack.

Motors

| Model | Manufacturer | Max HP | Min Voltage | Max Voltage | Cost | Vendor |
| --- | --- | --- | --- | --- | --- | --- |
| ES-67A-29 | Motor D & D | ? | 72 | 96 | $1395.00 | http://www.cloudelectric.com/inc/sdetail/1045 |
| ES-31B | Motor D & D | 90 | 72 | 144 | $1154.00 | http://www.cloudelectric.com/inc/sdetail/169 |
| #L91-4003 | Advanced DC | 72 | 72 | 120 | $1004.86 | http://www.evparts.com |

The 15kW shunt wound motor currently in the vehicle is completely insufficient. It will not meet the client’s needs. In fact, shunt motors are not used in EV’s because they have no low-end torque. This means that if you are parked on even a slight incline, the car will not move. A series wound DC motor is required to meet the client’s needs. This table above shows some suitable series wound DC motors.

Batteries

| Model | Manufacturer | Voltage | Cost | Vendor |
| --- | --- | --- | --- | --- |
| * 30XHS | Trojan | 12 | 163.15 | Safe-Start Contact George at (727) 423-7093 |
| **30XHS | Trojan | 12 | 195.95 | Jacksonville Battery Distributors Southeast, Inc. |

*This includes a university discount with the battery supplier

**Located in Jacksonville

2 Trojan Deep Cycle Batteries will be required to get the voltage from 72V to 96V. With the addition of the new controller there is room to later add 2 more batteries to the system to increase range and speed.

Cost Chart

| Motor -> Controller v | D&D ES-31B | D&D ES-67 | Advanced DC L91-4003 |
| --- | --- | --- | --- |
| Kelly KDH09500A | $2,279.30 | $2,520.30 | $2,130.30 |
| Kelly KDH12500A | $2,379.30 | $2,620.30 | $2,230.30 |
| Kelly KDH14600B | $2,625.30 | $2,866.30 | $2,476.30 |
| Curtis 1231C-7401 | $2,530.30 | $2,771.30 | $2,381.30 |
| Curtis 1231C-7701 | $2,877.30 | $3,118.30 | $2,728.30 |
| LogiSystems | $2,690.30 | $2,931.30 | $2,541.30 |

All prices include cost of 2 Trojan 30XHS batteries.

Options

| Option | New Motor | New Controller | Number of New Batteries | Configuration | Est. Top Speed | Est. Range* | Climb Hills Well? | Cost |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | N | N | 6 | Series/Parallel | ~55 MPH | ~60 Miles | No | $978.00 |
| 2 | Y | N | 0 | Series | ~60 MPH | ~35 Miles | Yes | $1004.86 |
| 3 | Y | N | 6 | Series/Parallel | ~70 MPH | ~60 Miles | Yes | $1982.86 |
| 4 | N | Y | 2 | Series | ~70 MPH | ~35 Miles | No | $971.00 |
| 5 | N | Y | 10 | Series/Parallel | ~65 MPH | ~70 Miles | No | $2330.00 |
| 6 | Y | Y | 2 | Series | ~70 MPH | ~40 Miles | Yes | $2030.00 |
| 7 | Y | Y | 10 | Series/Parallel | ~70 MPH | ~70 Miles | Yes | $3334.00 |

References

EV Album

www.evalbum.com

Trojan Batteries

www.trojanbattery.com

Curtis Instruments

www.curtisinstruments.com

EV Parts

www.evparts.com

EV Convert

www.evconvert.com

Kelly Controllers

www.kellycontroller.com