EML 3015C 	Thermal-Fluids I  	  Fall 2001				Lab-8

- Consider a Rankine cycle which uses steam as the working fluid (see figure below).  It is known that the pressure is 1 MPa, a constant, inside the boiler (2-3).  The steam leaves the boiler as 100% saturated vapor (state 3) when it enters the condenser.  Inside the condenser (4-1) the pressure is also a constant at 10 kPa.

Determine the thermal efficiency of the cycle.  (b) Determine the amount of heat rejection in the condenser.

Note: As we will discuss in Chapter 7, a Rankine Cycle is an ideal cycle for vapor power plants.  It has the following four processes:

- 1- 2 Isentropic compression in the pump

- 2- 3 Isobaric heat addition in the boiler

- 3- 4 Isentropic expansion in the turbine.

- 4- 1 Isobaric heat rejection in the condenser.

T

3

P2=P3=1 MPa

2

1

4

P1=P4=10 kPa

s

Qin

Boiler

Turbine

Compressor

(pump)

Heat exchanger

(Condenser)

1

2

3

4

Wout

Win

Qout