Homework Assignment  3  - EML 3015C Thermal-Fluids  I

## Note:  This is an individual assignment.  In addition to the problems below, also do the following from Fox & McDonalds, 4th ed.:  8.67, 8.83 & 8.88

## Due date: Friday, Sept. 27, 2002

- Liquid at 20 C enters heat exchanger in the solar panel collector and leaves at an unknown temperature To.  The mass flow rate is constant at 1 kg/s and Cp=2000 J/kg K (assumed to be constant within this temperature range).  It is known that 80% of the incoming solar irradiation of q"=2000W/m2 is absorbed by the solar panel.  The solar panel is exposed to the outside cold air at T=20 C with an averaged convection heat transfer coefficient of 30 W/m2 K (both sides). (a) Neglect emission radiation heat loss, determine the temperature of the liquid flowing out of the panel under steady state condition. (b) Do you think it is a good idea to neglect the emission radiation given =0.2, =5.67*10-8 (W/m2 K4), Tsurr=20 C?  Explain why or why not.

Answer: a) 26.3 C , b) q"=3.68 W/m2

Flow in

1 kg/s

Cp=2000 J/kg K

Tin=20 C

q"=2000 W/m2

Flow out

To=?

Cp=2000 J/kg K

T=20 C

h=30 W/m2 K

Solar panel

3 m by 3 m

- Water is draining out from a water tank as shown.  A reverse U-tube is used to measure the pressure difference between point 2 and point 3.  ZA-ZB=1 m.  Determine (a) the jet exit velocity, V4;  (b) The water level above the exit (z1-z4) at this instant; (c) The rate of decrease of the water level: dz1/dt; (For simplicity, use 5 m/s as the jet exit velocity). (d) The pressure of the air trapped inside the U-tube: PA or PB.  Patm=101,000 (N/m2), water=1,000 kg/m3.  Assume no losses.

Trapped air, low density

1

x

x

2

3

A

B

x

4

1 m

Tank cross-sectional area 1 m2

Small pipe area 0.01 m2

Large pipe area 0.02 m2

0.1 m

3.  A pump transfers water from one reservoir to another as shown.  The total frictional

loss of the piping is given as KL(V2/2g) where V is the averaged velocity in the pipe.

The total head provided by the pump ,H, is a function of the volume flow rate, Q, as

given by H=100-50*Q, where H in m of water, Q in m3/s.  If the loss coefficient

KL=10 and the pipe diameter is 0.1 m, what is the flow rate pumped to to the upper reservoir.

Answer:  0.0748 m3/s,

4. An adiabatic gas turbine which uses air as the working gas has air entering and leaving the turbine under the following conditions.

At the turbine inlet: P = 2 MPa, T = 1200 K, V = 50 m/s and z = 10m

At the turbine exhaust: P = 100 kPa, T = 600 K, V = 180 m/s and z = 6m

- Determine the work done per unit mass of air flowing through the turbine.

- Is it reasonable to neglect the change in kinetic  and potential energies in the analysis?  Please explain.

Note: For an ideal gas you may assume that  u = CvT and h = CpT

Answer:  657.6 kJ/kg