Objective:

Dynamic simulation of controller design for distillation columns in CHEMCAD

Outline:

Simulation in SCDS Dynamic column at steady state

Dynamic simulation in SCDS Dynamic column with Controller design

Data Available From Problem Definition:

- Feed flow rate

- Feed composition

- Feed temperature and pressure

- Desired specifications at top and bottom

Data Available From Controller Design in MATLAB:

- Proportional Gain K

- I

Facts:

- Proportional band=100/gain

- Increase in gain increases the speed of response

- No integral action will be taken in CHEMCAD when Ior I >1010

- Decreasing I gives a faster controller response but introduces a tendency to overshoot the set point producing oscillations.

- Derivative control anticipates the convergence on the set point so that any overshooting on oscillation is minimized.

Simulation Procedure for Dynamic SCDS column:

- Enter the feed stream characteristics

- Repeat the same procedure as that of the regular SCDS column for defining the specifications and column characteristics.

- First perform a steady state simulation to make sure that the system is feasible.

- Choose constant reboiler holdup and constant condenser holdup in the specs  option of the Column menu.

- Simulate the column in dynamic mode and make sure the specifications are met.

- Choose variable  reboiler holdup and variable  condenser holdup in the specs  option of the Column menu.

  - Choose Reflux specification as By control valve and enter Distillate rate obtained from previous simulations

  - Choose Bottoms  specification as Liquid bottoms mole rate  and enter Bottoms  rate obtained from previous simulations

- Choose Valve type as Linear valve with the following characteristics:

  - Critical flow factor 0.98

  - Supply pressure 2.5atm

  - Valve time constant 0.15

  - Valve Av 0.0625

  - Valve Bv –0.25

  - Valve flow coefficient 300

  - Rangeability 10

- Enter the controller design parameters obtained from MATLAB simulations in Controller

- Give a set point value for the reflux mole rate (measured variable) and select mole flow rate  as the measured variable  in the Sensor information

- Rest of the parameters on the reflux control option can be left blank.

- Simulate the dynamic system with the added controller information.

- Make adjustments to the tuning parameters as and when necessary.

- Plot the mole fractions of each component at the top (Distillate) using plot option.

- Repeat the procedure for 10% change on either side of the steady state values in Feed flow rate and feed composition and plot the distillate mole fractions with time.

- In each simulation, record the following information:

  - Feed stream conditions used

  - Simulation time and step size

  - Controller tuning parameters used

  - Variation of reboiler and condenser liquid holdup with time

  - Variation of measured variable (molar reflux flow) with time

Most of this information can be obtained by choosing Results on the menu bar and selecting the appropriate options.

- Compare the prediction of MATLAB simulations with the results obtained from CHEMCAD simulations