Objective:

Dynamic simulation of distillation columns in CHEMCAD

Outline:

Steady state simulation in shortcut column

Steady state simulation in Tower/SCDS column

Dynamic simulation in SCDS Dynamic column

Data Available:

- Feed flow rate

- Feed composition

- Feed temperature and pressure

- Desired specifications at top and bottom

Common Information:

Prior to all simulations in CHEMCAD, the following tasks must be performed:

- Select Engineering units

- Select components from components’ list

- Draw the flow chart from templates

- Add feed and product arrows wherever necessary

Shortcut Column:

- Input Temperature, Pressure, Feed flow rate and composition in feed stream

2.   Enter R/Rmin and product specification in Edit unit op data

- Simulate and obtain R calculated, Number of trays and feed tray, reboiler and

condenser heat duties

Tower:

- Input Temperature, Pressure, Feed flow rate and composition in feed stream

- Double click on the unit operation.

- Select condenser type (total condenser) and enter number of trays and feed tray location as the General model parameters

- In the Specifications tab, enter the given specifications for reboiler and condenser

- The Convergence and Cost Estimation tabs are optional and can be left blank for our purposes

- Enter OK

- Go to Run menu and select convergence. Find the block that gives the option steady state/dynamics (on the bottom left hand side corner).

- First select steady state and simulate the column similar to the short-cut column.

- Check the results of the simulations to see if they compare with the short-cut column predictions

- Now, go to Run menu again and select convergence. This time, pick dynamics.

- Go to Run menu and click on dynamics to begin the dynamic simulation procedure.

- Select the first item, set run time and a new window, Dynamic run time schedule appears. Go to the step1 tab and select run time and step size

- Use the items, Record streams, Record unit operations, Record process to record necessary information during the simulation.

- Use the item, Run from initial state to complete the simulation. Plot the necessary information.

SCDS Column:

- Steps 1 through 6 are the same as those of Tower.

- In the dialogue box titled Column menu, use the first item Initial condition. Choose steady state continuous process and ignore tray vapor holdup. Choose Variable pressure.

- Enter column holdup in the dialogue box.

- Choose constant holdup for reboiler and condenser during the simulations. Select control valve for condenser holdup only during controller design (procedure will be provided for controller design separately).

- Use the same method as that of Tower to simulate the column at steady state as well as for the dynamic simulation.

Example to be discussed in class:

Feed: 300kmols of 50% Toluene and 50% O-Xylene at 330K and 1.2 atm

Specifications: 88%Toluene and 12%O-Xylene at the top

Next topic:

Simulation of PID controller using SCDS dynamic column