CSTR Example:

Reaction:

Decomposition of di-t-butyl peroxide (DTBP) to acetone and ethane

(CH3)3COOC(CH3)3→2(CH3)2CO+C2H6

Reaction rate:

Reaction conditions:

Temperature 154.6 oC

Pressure 491.8 mmHg

Feed and feed conditions:

100 kmol/h of (DTBP) at 110 oC and 760 mmHg

Information for steady state:

- Reaction is isothermal

- Conversion with respect to DTBP is 85%

- Reference temperature for heat of reaction is 25 oC

Additional specifications for dynamic simulations:

- Simulate the reactor for 100 min with 5min as step size

Problem Statement:

- Perform the steady state simulation. What is the required volume of the reactor at steady state? What is the steady state heat duty? What are the steady-state mole rates of components in the product stream?

- Perform dynamic simulation with no disturbance.

- Perform dynamic simulation with the following feed characteristics

  - 0 to 20 min: 100 kmol/hr

  - 20 to 40 min: 90 kmol/hr

  - 40 to 60 min: 110 kmol/hr

  - 60 to 100 min: 100 kmol/hr

and plot the feed flow rate with time.

- Perform dynamic simulation with 10 kmol/hr of acetone in addition to 100 kmol/hr of DTBP in the feed.

- For items 2 through 4, report the required reactor volume and heat duty.

- Design a PID controller to keep the product component mole rates at their corresponding “steady state” values despite the disturbances in feed flow rate as described in items 3 and 4. Plot the product mole rates with time for each case.

- Provide recommendations to use the simulator more efficiently for the dynamic simulation and control of CSTR. (Hint: Any additional information to be given the simulator, other reactor characteristics worth observing during simulation, shortcomings in the unit operation used, etc.)