CHEMICAL PROCESS CONTROL: PROJECT

A 20-tray distillation column is used to separate the binary mixture of acetone and methanol. Equimolar mixture of 200 kmol/h enters the column at the 10th stage at 250 K and 0.5 atm. The top pressure of the column is 0.3 atm while the pressure at the reflux pump is 1.5 atm.

Notes:

- Use UNIFAC method for K-Values and Latent Heat method for Enthalpy for all sections of the project.

- Always run the simulation at steady state before running a new or edited dynamic simulation.

- Submit only the tables, plots and snap shots requested under each section of the problem statement. Additional plots DO NOT carry extra points!

Section-1 Steady State Simulations:

1. Perform a steady state simulation of this distillation column under various specifications for condenser and reboiler to complete Table 1.

2. Repeat Problem 1 in this section for a 200 kmol/h feed with 62.5% acetone and 37.5% methanol.

Report completed Table 1 for both of the above scenarios.

No plots are required in this section!

Section-2 Dynamic Simulation:

Perform a dynamic distillation simulation with the following characteristics for 400 minutes:

The feed to the column is 200 kmol/h equimolar mixture of acetone and methanol up to the first 25 min. A step change is given to the feed at 25 min where the mole fraction of acetone and methanol are ramped to 0.625 and 0.375 respectively while the total flow rate still remains at 200 kmol/h. Use the reflux mole rate and reboiler heat duty from first row of Table 1 (calculated for problem-1 under Section-1) as the specifications for this dynamic simulation. Use negligible condenser holdup, 0.1 kmol as tray hold up and 10 kmol for reboiler hold up in the simulation.

Report the following:

- Snap shot of flowsheet layout used for simulation

Plot the following parameters with time:

- Mole fractions of acetone and methanol in the feed stream

- Distillate total mole rate

- Mole rate of acetone in distillate

- Bottom total mole rate

Section-3 Controller Design:

1. For the dynamic simulation described in Section-2, design a control valve coupled with a PID controller to maintain the mole rate of acetone in distillate at 99 kmol/h despite the changes in feed to the column. Use a valve time constant of 0.15 and a downstream pressure of 0.3 atm for the control valve.

Report the following:

- Snap shot of flowsheet layout used for the simulation

- Snap shot of control valve specifications page

- Snap shot of PID controller specifications page

Plot the following parameters with time:

- Mole rate of acetone in distillate

- Mole rate of liquid distillate

- Controller error

- Control valve opening percentage

- Distillation column top mole holdup (condenser holdup)

2. For the controller designed in problem 1 of this section, find the corresponding Ziegler-Nichols tuning parameters by modifying the tuning parameters (PB, Taui and Taud).

Report the following:

- Snap shot of controller specifications page (with the PB, Taui and Taud used to calculate the Ziegler-Nichols tuning parameters)

- Plot of acetone mole rate in distillate with time used to calculate the Ziegler-Nichols tuning parameters

- Snap shot of controller specifications page (with PB, Taui and Taud obtained from Ziegler-Nichols tuning parameters)

- Plot of acetone mole rate in distillate with time using the Ziegler-Nichols tuning parameters calculated above.

Table 1: Steady-State Simulation Results

| # | Condenser Specification |  | Reboiler Specification |  | Moles of Acetone in Distillate | Reflux Mole Rate | Distillate Mole Rate | Reboiler Heat Duty | Vapor to Bottom Flow Ratio | Bottom Mole Rate | Condenser Heat Duty |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Specification | Value | Specification | Value |  |  |  |  |  |  |  |
| 1 | Dist. Rate of Acetone | 99 kmol/h | Reboiler Duty | 20,000 MJ/h |  |  |  |  |  |  |  |
| 2 | Dist. Rate of Acetone | 99 kmol/h | V/B Ratio | 4 |  |  |  |  |  |  |  |
| 3 | Dist. Rate of Acetone | 99 kmol/h | Bottom Total Flow | 80 kmol/h |  |  |  |  |  |  |  |
| 4 | Reflux Mole Rate | 300 kmol/h | Bottom Total Flow | 80 kmol/h |  |  |  |  |  |  |  |
| 5 | Reflux Mole Rate | 300 kmol/h | Reboiler Duty | 15,000 MJ/h |  |  |  |  |  |  |  |
| 6 | Reflux Mole Rate | 300 kmol/h | V/B Ratio | 2 |  |  |  |  |  |  |  |
| 7 | Condenser Heat Duty | -15000 MJ/h | Bottom Total Flow | 80 kmol/h |  |  |  |  |  |  |  |
| 8 | Condenser Heat Duty | -11000 MJ/h | V/B Ratio | 3.5 |  |  |  |  |  |  |  |
| 9 | Distillate Mole Rate | 100 kmol/h | V/B Ratio | 2.5 |  |  |  |  |  |  |  |
| 10 | Distillate Mole Rate | 90 kmol/h | Reboiler Duty | 18,000 MJ/h |  |  |  |  |  |  |  |

Condenser holdup negligible in Section-2 only. This is not necessarily negligible in Section-3.