ECH 4323: CHEMICAL PROCESS CONTROL:

FINAL PROJECT

A 20-tray distillation column is used to separate the binary mixture of acetone and methanol. The feed containing 100 kmol/h of acetone and 100 kmol/h of methanol enters the column at the 10th stage at 250 K and 0.5 atm. The top pressure of the column is 0.3 atm while the pressure at the reflux pump is 1.5 atm.

Notes:

- Use UNIFAC method for K-Values and Latent Heat method for Enthalpy for all sections of the project.

- Always run the simulation at steady state before running a new or edited dynamic simulation.

- Submit only the tables, plots and snap shots requested under each section of the problem statement. Additional plots DO NOT carry extra points!

- Write a report that clearly describes what you did.  Use your experience from the unit operations lab to write this report.

Section-1 Steady State Simulations:

1. Perform a steady state simulation of this distillation column under various specifications for condenser and reboiler to complete Table 1.

2. Repeat Problem 1 in this section when the feed consists of 125 kmol/h of acetone and 75 kmol/h of methanol.

Report completed Table 1 for both of the above scenarios.

No plots are required in this section!

Section-2 Dynamic Simulation:

Perform a dynamic distillation simulation with the following characteristics for 400 minutes:

The feed to the column contains 100 kmol/h of acetone and 100 kmol/h of methanol up to the first 25 min. A step change is given to the feed at 25 min where the acetone flow rate becomes 125 kmol/h and and the methanol flow rate becomes 75 kmol/h (the total flow rate still remains at 200 kmol/h). Use the reflux mole rate and reboiler heat duty from first row of Table 1 (calculated for problem-1 under Section-1) as the specifications for this dynamic simulation. Use negligible condenser holdup, 0.1 kmol as tray hold up and 10 kmol for reboiler hold up in the simulation.

Report the following:

- Snap shot of flowsheet layout used for simulation

Plot the following parameters with time:

- Mole fractions of acetone and methanol in the feed stream

- Distillate total mole rate

- Mole rate of acetone in distillate

- Bottom total mole rate

Section-3 Controller Design:

1. For the dynamic simulation described in Section-2, design a control valve coupled with a PID controller to maintain the mole rate of acetone in distillate at 99 kmol/h despite the changes in feed to the column. Use a valve time constant of 0.15 and a downstream pressure of 0.3 atm for the control valve.

Use the Ziegler-Nichols tuning method to design the following controllers:

- P controller

- PI controller

- PID controller

Report the following:

- Snap shot of flowsheet layout used for the simulation

- Snap shot of control valve specifications page

- Snap shot of PID controller specifications page

- Snap shot of controller specifications page (with the PB, Taui and Taud used to calculate the Ziegler-Nichols tuning parameters)

- Plot of acetone mole rate in distillate with time used to calculate the Ziegler-Nichols tuning parameters

Plot the following parameters with time:

- Mole rate of acetone in distillate

- Mole rate of liquid distillate

- Controller error

- Control valve opening percentage

- Distillation column top mole holdup (condenser holdup)

Table 1: Steady-State Simulation Results

| # | Condenser Specification |  | Reboiler Specification |  | Moles of Acetone in Distillate | Reflux Mole Rate | Distillate Mole Rate | Reboiler Heat Duty | Vapor to Bottom Flow Ratio | Bottom Mole Rate | Condenser Heat Duty |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Specification | Value | Specification | Value |  |  |  |  |  |  |  |
| 1 | Dist. Rate of Acetone | 99 kmol/h | Reboiler Duty | 20,000 MJ/h |  |  |  |  |  |  |  |
| 2 | Dist. Rate of Acetone | 99 kmol/h | V/B Ratio | 4 |  |  |  |  |  |  |  |
| 3 | Reflux ratio | 1 | Bottom Total Flow | 80 kmol/h |  |  |  |  |  |  |  |
| 4 | Reflux Mole Rate | 300 kmol/h | Bottom Total Flow | 80 kmol/h |  |  |  |  |  |  |  |
| 5 | Reflux Mole Rate | 300 kmol/h | Reboiler Duty | 15,000 MJ/h |  |  |  |  |  |  |  |
| 6 | Reflux Mole Rate | 300 kmol/h | V/B Ratio | 2 |  |  |  |  |  |  |  |
| 7 | Condenser Heat Duty | -15000 MJ/h | Bottom Total Flow | 80 kmol/h |  |  |  |  |  |  |  |
| 8 | Condenser Heat Duty | -11000 MJ/h | V/B Ratio | 3.5 |  |  |  |  |  |  |  |
| 9 | Distillate Mole Rate | 100 kmol/h | V/B Ratio | 2.5 |  |  |  |  |  |  |  |
| 10 | Distillate Mole Rate | 90 kmol/h | Reboiler Duty | 18,000 MJ/h |  |  |  |  |  |  |  |

Condenser holdup negligible in Section-2 only. This is not necessarily negligible in Section-3.