EML 3013	Dynamic Systems I		Lab-2			Spring 2007

(Turn in parts 2-A & 2-B next Friday; turn in part 2-C with your lab report-experiment 2)

Lab-2A. (also 13-96 from the textbook) The forked rod is used to move the smooth 2-lb particle around the horizontal path in the shape of a limaçon, r = (2 + cos θ) ft. If θ = (0.5t2) rad, where t is in seconds, determine the force which the rod exerts on the particle at the instant t = 1 s. The fork and path contact the particle on only one side.

Lab-2B. It can be shown that the angle, , between the extended radial line from a cylindrical coordinate to the tangent of the curve can be given by

tanψ=rdrdθ, where r is the radial distance from the origin and θ is the angular positonof the cylindrical coordinate.

Plot the shape of the limaçon in part 2-A, that is r=2+cosθ in the range 0≤θ≤2 (rad).  Determine numerically the angle using an angular increment of 0.05 radian.  Compare your numerical values with the theoretical values obtained by the given formula.

Lab 2-C.  Cart A is being dragged along a frictionless surface as shown.  Assume the force is a constant 100 N, (1) determine the acceleration aA as a function of the displacement x.  (2) Also, determine the velocity V of the cart when it reaches just underneath the pulley.  (3) How can we approach the same problem if the constant F is replaced by a free hanging weight with a mass of 10 kg (it produces an equivalent 98 N of force).  Derive the relationship and discuss.  We will tackle this together in the class.

mA=10 kg

x

10 m

10 m

F=98 N

Cases (1) & (2)

10 kg

Case (3)