EML 3013		Dynamic Systems I		Lab-4			Spring 2007

Due March 1, 2007 in lab period

The ball has a mass of 1 kg is attached to a weightless cord (2 m long).  It is released from rest from =0° until the string strikes a horizontal rod placed at the middle of the cord when =30° as shown.  Determine

- The speed of the ball when the ball reaches its lowest point.

- The tension along the cord.

- The force acting on the horizontal rod.

You have to calculate this problem using two approaches:

  - Using the tangential/normal equation of motion and integrate with respect to  first, then with respect to 

  - Using the concept of work and energy.

2 m

1 m



