EML 3013	example					Spring 2007

A hydraulic jack is used to lift up the ramp with an incline angle  as shown.  Determine when the block (10 kg) will (a) slip, (b) tip, (c) both.  The static and kinetic frictions between the block and the truck bed S= 0.5, k=0.4 and the moment of inertia of the block IG=10 kg.m2 with respect to its center of gravity (also the geometric center of the block).



1 m

2.2 m

S=0.5 k=0.4

y

x

x

F

W

N=Wcos()

MT, Tipping moment by friction force F

MC, Counter balance moment by shifting normal force N

ab

(a)  to make  sure  no slipping⇒x−force  required: Fμ=Wsinθy−force: N=WcosθThe maximum angle: Fμ=Wsinθ=μSN=μWcosθ, μS=tanθWhen θ>26.56° slipping will occureMT=Fμ(2.22)=(Wsinθ)(1.1)=(1.1)WsinθMC=Nx=(Wcosθ)(x), MC=MT⇒x=1.1tanθ, when θ>24.44°, x>0.5 mhence tipping will occur when the angle is set between 24.44° and  26.56°

Let use the angle of 25° as an example to illustrate what will happen:No slipping but there is a net tipping moment:MG=IGα⇒Fμ(2.22)−N(0.5)=(10)(9.8)sin(25°)(1.1)−(10)(9.8)cos(25°)(0.5)MG=5.7=IGα=(10)α, α=0.57(rads2)One can than determine the acceleration of the center of the mass of the blockaG=at+α×rGt−ω2rGt=0+(0.57)k×0.5i+1.1j=0.285 {j−0.627 {i(ms2)this is the initial acceleration when ω=0

(b) What will happen if the inclination is suddenly increased beyond 26.56° and theblock does not get the chance to tip over yet?It is known that the block will slide but not necessarily tip over since the frictional force will changeFμ=μkN=(0.4)WcosθThe new tipping moment: MT=Fμ(2.22)=(0.44)WcosθMC=Nx=Wcosθ(x), MC=MT⇒x=0.44m⇒ have enough counter balance momentto prevent tipping The block will slide down but not tip over