EML 3015C Thermal Fluids I				Quiz 3		fall 1999

A throttling device is a conduit with a narrow throat passage as shown in the figure.  A U-tube containing water is connected from the inlet to the throat in order to measure the pressure difference between these two points.  The water level at points A & B is level when no airflow passes through.   What will happen to the water level A and B when a strong airflow is blowing through the throttle device?  Air density is much lower compared to the water; therefore, the pressure inside the tube is the same as that measured at the water free surface in the U-tube.  That is, P1=PA, and P2=PB.  Bernoulli’s equation: Pρ+V22+gZ=constant

1

2

A

B

From mass conservation: A1V1=A2V2, V2>>V1From Bernoulli's Equation: P1ρ+V122+gz1=P2ρ+V222+gz2, therefore, P1>P2P1=PA>PB=P2.   Consequently, the level at B will rise and the level at A will drop.