EML 3015 C Thermal-Fluid I, Sample Final Exam 2				fall 1999

- A square 10 m by 10 m flat-plate solar collector is used to collect solar heat flux to generate hot water.  It is known that 80% of the 1000 W/m2 solar flux is absorbed by the working fluid.  (a) Determine the mass flow rate of the system so that the water temperature increases from 20C to 40C flowing through the collector (Cp=4 kJ/kg.K).  (b) Assume the average temperature of the top surface is 30C and the bottom surface is well-insulated.  What is the total radiation heat loss due to surface emission? The surface emissivity () is 0.5 and the surrounding temperature is 20C? Boltzmann constant =5.67*10-8 (W/m2 K4).  (c) It is known that 10% of the solar flux reflects off the collector, what is the averaged convection coefficient of the collector surface?

20C

40C

1000 W/m2

T=20C

Tsurr=20C

10% reflection

2. Two water hoses are used to fill the tank as shown below.  In one hose, hot water at 80C is flowing into the tank at a mass flow rate of 1 kg/s and in the other, cold water at 20C is flowing at a rate of 1.5 kg/s. (Assume constant thermal properties: density =1000 kg/m3, Cp=4 kJ/kg.K)  (a) Determine the rate of increase of the level of the water inside the tank.  (b) How long will it take to fill the tank to a water level of 1 m if starting from empty? (c) What is the water temperature at that instant? (d) If one stops the filling process and opens a draining valve (0.001 m2 in opening area) at the bottom of the tank, what will be the exiting flow velocity at this instant? (neglect friction and entrance losses) (e) If the valve is opened, how long will it take to completely drain the tank (starting from 1 m water height)?

H

C

Aexit=0.001 m2

Tank cross-sectional area 0.1 m2

3. 20 kg/s Steam enters the turbine at 400C and 8 MPa and exits as a mixture of saturated vapor and liquid at a pressure of 0.1 MPa.  It is known that the turbine delivers 8 MW power.  (a) Determine the isentropic efficiency of the turbine.  (b) What is the actual quality of the steam when it exits from the turbine.

Steam 20 kg/s, 400C and 8 MPa

8 MW work output

Saturated steam

0.1 MPa

4. Inside a condenser unit, saturated steam at 0.1 MPa flows inside a circular iron pipe with an inside diameter of 2 cm and an outside diameter of 2.5 cm (k=80 W/m.K).  As shown in the figure, outside the steam pipe, cooling water enters at a temperature of 20C and leaves at an unknown temperature TO.  (a) Estimate the average heat flux (heat rate per unit area) passing through the steam pipe wall if the cooling water flow rate is 200 kg/s.  (b) If the pipe is 5000 m long, what is the output temperature of the cooling water? (c) If all saturated vapor converts into saturated liquid inside the condenser unit, what is the mass flow rate of the steam?  Assume convection coefficients inside and outside the pipe are h1=200 W/m2.K and h2=50 W/m2.K, respectively.

Saturated vapor (100% quality)

0.1 MPa

Water in, 20C

Water out,

TO

T,1

T,2

Saturated liquid (0% quality),

0.1 MPa

Entropy balance analysis: dsdt=QT+m1s1−m2s2+sgenerationTds equation: Tds=du+Pdv=dh-vdPEntropy change of an idea gas:s2−s1=12cv(T)dTT+Rlnv2v1 or s2−s1=12cP(T)dTT−RlnP2P1For a reversible process:Q=TdS,    W=PdV

Ideal gas: PV=mRT

H(enthalpy)=mCpT, U(energy)=mCvT

Adiabatic (isentropic) compression/expansion process for an ideal gas going from 1 to 2: