EML 3015C	Thermal Fluids I		Test 1				fall 1999

- Water is draining out from a water tank as shown.  A reverse U-tube is used to measure the pressure difference between point 2 and point 3.  ZA-ZB=1 m.  Determine (a) the jet exit velocity, V4;  (b) The water level above the exit (z1-z4) at this instant; (c) The rate of decrease of the water level: dz1/dt; (For simplicity, use 5 m/s as the jet exit velocity). (d) The pressure of the air trapped inside the U-tube: PA or PB.  Patm=101,000 (N/m2), water=1,000 kg/m3.  Assume no losses.

A

Trapped air, low density

1

1 m

B

4

0.1 m

x

x

x

2

3

Small pipe area 0.01 m2

Tank cross-sectional area 1 m2

Large pipe area 0.02 m2

(a) P2ρ+V222+gz2=P3ρ+V322+gz3, z2=z3, From mass conservation: A2V2=A3V3V3=A2A3V2=2V2, 1ρ(P2−P3)=12(V32−V22)=32V22P2=PA+ρghA−2, P3=PB+ρghB−3, P2−P3=ρg(hA−2−ρghB−3)=ρghA−BV2=23ghA−B=2.56(ms), V3=5.11(ms)(b) V4=V3=5.11(ms)From Bernoulli's equation: g(z1−z4)=12V42, z1−z4=1.33(m)(c) dmdt=ρAtdz1dt=−ρA4V4, dz1dt=−A4AtV4=−0.011(5)=−0.05(ms)(d) P2ρ+V222+gz2=P1ρ+V122+gz1, P2=P1+ρg(z1−z2)−ρ2V22P2=PA+ρghA−2, PA=P2−ρghA−2=P1+ρg(z1−z2)−ρghA−2−ρ2V22PA=P1+ρg(1.33)−ρg(1.1)−ρ2(2.56)2101000+(1000)(9.8)(0.23)−10002(2.56)2=99,972(Pa)

- One side of the space station is exposing to the sun and receives a uniform solar irradiation of 3,000 W/m2 as shown.  The space station wall is made of a special metal alloy with a thickness of 0.1 m and a thermal conductivity k=20 W/m K.  Inside the station, the temperature is maintained at a comfortable 20C and the convective coefficient is 30 W/m2 K.  Assume steady state, negligible radiation emission, and the outside wall temperature is measured to be TO=100C, (a) what is the inside wall temperature TI?  (b) What is the reflectivity of the outer surface? (Assume the surface is opaque - not transparent)  (c) For this part, assume that =0.7 and the inside wall surface stays at a constant temperature of 80C (note: these might not be the answer for parts (a) & (b)), now if we include the radiation emission from the outside surface, which has a surface emissivity of 0.2, what should be the outside wall temperature TO?  Assume the outer space temperature is 20 K and the direction of the heat transfer is one-dimensional.  (Note: The outer space can be assumed to be vacuum). =5.67x10-8 W/m2 K4.

TI: inside wall temperature

Solar irradiation G=3,000 W/m2

T

Tsurr=20K

Inside the station, T=20 C, h=30 W/m2 K

TO: outside wall temperature

  - m thick

k=20 W/m K

(a) Steady state: qcond=qconv, kTO−TIL=h(TI−T∞)(20)100−TI0.1=(30)(TI−20), TI=89.6(°C)(b) h(TI−T∞)=(30)(89.6−20)=irradiation absorbedαG=α(3000), α=0.696ρ=1-α−τ=1-0.696=0.304(c) (0.7)(3000)=radiation emission (qrad)+ conduction through wall (qcond)εσ(TO4−204)+kTO−(80+273)0.1Assume negligible radiation: 2100=(20)TO−3530.1, TO=363.5(K)Radiation: qrad≈(0.2)(5.67×10−8)(363.54−204)=198(Wm2)Correction: 2100-198=1902=qcond=k(TO)correction,1−(80+273)0.1(TO)correction,1=362.5(K)qrad≈196(Wm2)(TO)correction,2=362.52(K) Good enough

A rectangular-shaped hydraulic system is used to move a heavy object as shown.  A 100 N force is pushing down against piston A.  (a) What is the gage pressure (P-Patm) at the center of the piston B?  (b) What is the total force acting on the heavy block by the piston B, FB.  Neglect weighs of both pistons.

FA=100 N

Piston A

Area 0.05 m2

Oil

=800 kg/m3

1

2

FB=?

0.5 m

0.2 m

0.5 m

Piston B

Area 0.5 m2

Water

=1000 kg/m3

(a) PAAA=FA+PatmAAPA=FAAA+PatmAt the center of the piston B: PB=PA+ρoilghA−1+ρwatergh1−2+ρwatergh2−BPB,gage=PB−Patm=(PA−Patm)+(800)(9.8)(0.5)+(1000)(9.8)(0.2+0.25)FAAA+3920+4410=10,330(Pa)(b) The pressure is not a constant along the surface of the piston B; the pressure isa function of its depth y measured below level 2P(y)=P2+ρwatergy, P2=PA+ρoilghA−1+ρwatergh1−2=7,880(Pa)P(y)=7880+(1000)(9.8)yFB=PdA=P(y)(1)dy, Piston B has a width of 1 mFB=(7880+9800y)dy=7880(0.5−0)+9800y2200.5=5,165(N)Note: The force can also be estimated using FB=PB,gageAB=(10330)(0.5)=5,165(N)since Paverage,gage=PB,gage.  However, this might not be the case for all static conditions aswe will learn later.

Extended Bernoulli's Equation:P1ρg+V122g+z1+hA−hE−hL=P2ρg+V222g+z2, hL=KLV22g, KL: loss coefficient