EML 3015C		Quiz 4					fall 1999

(Process-1) Liquid water is being heated up in a closed piston-cylinder assembly as shown.  Initially, the heating process is taking place at a constant pressure of 0.8 MPa until it reaches the saturated condition: that is, all liquid water becomes saturated liquid water.

(a) What is the temperature at this instant?

(Process-2) After the system reaches the saturated condition, the piston is locked with a pin (not moving) and heat is removed to allow the system to cool down until its pressure reaches 0.1 MPa.

(b) What will be the temperature at this instant?

(c) What is the quality (x) of the saturated mixture? v=(1-x)vf+xvg

(d) On the T-v diagram below, draw a line showing the path taken by process 2.

Constant pressure line P=0.8 MPa

A constant weigh

T

Not to scale

Locking pin

Constant pressure line P=0.1 MPa

Process 2

v

Heat

- From the saturated water table, Tsat=170.43C when Psat=0.8 MPa, vf,1=0.001115(m3/kg).

- Since the piston is locked, the volume of the container will not change, therefore, the specific volume will not change also.  v2=vf,1.  From the saturated water table, Tsat=99.63C when Psat=0.1 MPa, vf,2=0.001043(m3/kg), vg,2=1.694(m3/kg).

- vf,1=v2=(1-x)vf,2+xvg,2,  0.001115=(1-x)(0.001043)+x(1.694), x=4.25x10-5,  very low quality means most of the mixture is in liquid form.