EML 3015C Thermal-fluids I	 Make-up Test #3				Nov. 19, 1998

- (45%)Considering the ideal Rankine power plant as shown. Steam enters the first-stage turbine at 8 MPa, 480C, and expands to 1 MPa.  It is then reheated to 400C before entering the second-stage turbine, where it expands to the condenser pressure of 0.008 MPa.  The net power output is 100 MW.  Determine (a) the thermal efficiency of the cycle, (b) the mass flow rate of steam, in kg/s, (c) the rate of heat transfer from the condensing steam as it passes through the condenser, in MW.

1

T

6

1

2

3

1st turbine

boiler

2

3

2nd turbine

condenser

6

pump

4

5

4

S

2. Two insulated tanks are connected by a valve.  One tank initially contains 5 kg of air at 80C, 0.1 MPa, and the other contains 1 kg of air at 50C, 0.2 MPa.  The valve is opened and the two quantities of air are allowed to mix until equilibrium is attained.  Use ideal gas assumption, determine (a) the final temperature, (b) the final pressure, (c) the amount of entropy produced.

3. 1 kg of air undergoes a cycle consisting of three reversible processes in series: 1-2: isothermal compression from 0.1 MPa to 0.475 MPa.  2-3: constant pressure expansion to temperature 390 K.  3-1: adiabatic expansion to the initial state.  Use ideal gas model, (a) sketch the P-v and T-s diagrams, (b) determine the work done and the heat transferred for each process.

Formula Sheet

Entropy balance analysis: dsdt=QT+m1s1−m2s2+sgenerationEnergy equation: dEdt=dEgdt+Q+E1−E2−dWdt, E=m(h+V22+gz)Tds equation: Tds=du+Pdv=dh-vdPEntropy change of an idea gas:s2−s1=12cv(T)dTT+Rlnv2v1 or s2−s1=12cP(T)dTT−RlnP2P1Isentropic process of an ideal gas:T2T1=v1v2k−1, and T2T1=P2P1k−1kand P2P1=v1v2k, where k=cpcvFor a reversible process:Q=TdS,    W=PdV