EML 3015C Thermal-Fluid I		Quiz 7			fall 1999

Steam (at a pressure of 2 MPa) exiting from the boiler expands adiabatically through the turbine (into condenser at a lower pressure of 10 kPa) as shown.  (This process 1-2 can be assumed to be an isentropic process, s=constant)  In order to avoid having liquid water in the turbine, the steam is superheated to a temperature T1 and then expands to become 100% saturated vapor (state 2) before entering the condenser.  (a) Determine the temperature T1 (you don’t need interpolation, get an approximated value will be sufficient); (b) determine the work output per unit mass (kJ/kg) from the turbine W12.

T

Constant pressure line P=2 MPa

1

T1

Constant pressure line P=10 kPa

2

s

(a) P=10 kPa, sg=8.1502 kJ/kg K= s1, h2=hg=2584.7 kJ/kg

From superheat table, P1=2 MPa, s1=8.1502 kJ/kg K, T1800 C

h1=4150.3 kJ/kg

(b)	W12=h1-h2=1565.6 kJ/kg