EML 3015C Thermal-Fluids I		Assignment 7-2		fall 1999

A 1 m3 (V1) piston/cylinder assembly is filled with 1 kg of air at 100 K (T1).  The system is undergoing through a complete cycle as shown.  It is known that from 1 to 2, the system is well insulated and there is no heat transfer (adiabatic process).  The volume is compressed to V2=0.5 m3.  From 2 to 3, a locking pin is used to fix the piston and allows the system to cool down to it original temperature (T3=T1).  Finally, the pin is released and the piston expands and returns to its original position.  (For air, R=0.287 kJ/kg K, Cv=0.718 kJ/kg K, k:specific heat ratio =1.4).  Determine (a) the temperature T2 after the adiabatic compression, (b) the amount of internal energy change, U1-2, (c) the amount of work: W1-2.  Also, (d) what is the work done from 2 to 3, W2-3?  (e) What is the heat transfer, Q2-3? (f) What is the internal energy change from 3 to 1, U3-1?  (g) What is the work done from 3 to 1, W3-1? (h) What is the heat transfer from 3 to 1, Q3-1?

Energy balance (closed system): Q-W=U

For an ideal gas: PV=mRT, U=mCvT,

Ideal gas undergoing adiabatic process: temperature change is related to the volume change according to the relationship:TATB=VBVAk−1

T

2

1

3

V