EML 3015C Thermal-Fluids I 		Test #1				Fall 1998

- Cold liquid at 20 C enters the solar panel collector and leaves at an unknown temperature To.  The mass flow rate is constant at 1 kg/s and Cp=2000 J/kg K (assumed to be constant within this temperature range).  It is known that 80% of the incoming solar irradiation of q"=2000W/m2 is absorbed by the solar panel.  The solar panel is exposed to the outside cold air at T=20 C with an averaged convection heat transfer coefficient of 20 W/m2 K (both sides). (a) Neglect emission radiation heat loss, determine the temperature of the liquid flowing out of the panel under steady state condition. (a) Do you think it is a good idea to neglect the emission radiation given =0.2, =5.67*10-8 (W/m2 K4), Tsurr=20 C?  Explain why or why not.

Flow in

1 kg/s

Cp=2000 J/kg K

Tin=20 C

q"=2000 W/m2

Flow out

To=?

Cp=2000 J/kg K

T=20 C

h=30 W/m2 K

Solar panel

3 m by 3 m

2. Block A, with unknown weight, is supported by the container filled with water and oil.  If the U tube is open to the atmosphere at one end, (a) determine the weight of block A.  (b) If an additional 10% of the weight is added to the block, how much more the mercury column will rise up from position 4 as shown?

Oil

oil=800 kg/m3

Block A

Area 1 m2

WA=?

0.8 m

Mercury

Hg=13500 kg/m3

Water

water=1000 kg/m3

1

2

3

4

0.5 m

0.5 m

0.2 m

3. (a) Determine the pressure at section 2.  (b) Determine the rate of decrease of the water level, that is, to find dh/dt.  P=101000(Pa).

h=2 m

Section 2

Area 0.03 m2

Water

=1000 kg/m3

Section 3

Exit Area 0.05 m2

Not to scale

Tank area 5 m2

Section 1

Free surface