EML 3015C		Quiz 5					fall 1999

Liquid water is being heated up in a closed piston-cylinder assembly.  Initially, the heating process is taking place at a constant pressure of 0.4 MPa until all liquid vaporizes into 100% saturated vapor (state 1).

(a) What is the specific volume (m3/kg) at this instant?

After the system reaches state 1, the piston is locked with a pin (piston not moving) and heat is removed to allow the system to cool down until its pressure reaches 0.1 MPa (state 2).

(b) What is the quality (x) of the saturated mixture at state 2? v=(1-x)vf+xvg

The pin is removed after the system reaches state 2 to allow the piston to freely expand.  Heat is again provided to vaporize all saturated mixture into 100% saturated vapor (state 3).

(c) What is the specific volume of the steam at state 3?

Finally, the locking pin is use to lock the piston at state 3 and heat is continuously added to the piston to heat the system back to the original pressure 0.4 MPa (state 4).

(d) What is the temperature at state 4? (Estimation of an approximated value will be enough)

(e) Draw the path (processes from 1 to 4: 1-2-3-4) taken by the system.

Not to scale

Constant pressure line P=0.4 MPa

A constant weigh

T

State 1

Locking pin

Constant pressure line P=0.1 MPa

4

2

3

v

Heat

- Saturated vapor at 0.4 MPa, vg,1=0.4625(m3/kg)

- From 1 to 2, constant specific volume process, v2=vg,1=0.4625(m3/kg)

From saturated steam table, at 0.1 MPa, vf,2=0.001043(m3/kg), vg,2=1.694(m3/kg)

v2=(1-x)vf,2+xvg,2, 	(0.4625)=(1-x)(0.001043)+x(1.694), x=0.273

- At state 3, pressure=0.1 MPa, v3=vg,3=1.694(m3/kg)

- From state 3 to state 4, the specific volume is again a constant. v4=v3=vg,3, from the superheated table, at a pressure of 0.4 MPa, the temperature should be approximately equal to 1,200C.

- See the red line 1-2-3-4 in graph