EML 3015C Thermal-Fluid I 	sample Final Exam 1					fall 1999

Note: show all necessary steps to receive full credit.

1. (25%) Consider a Rankine power plant as shown.  For both turbines, solid line represents the actual processes of the cycle, while the dash line representing the idealized isentropic expansion processes.  (Note: States 3s and 5 fall on the saturated vapor line.)  The steam is heated in the boiler at a constant pressure of 10 MPa to 550C and expands in the first turbine from state 2 to state 3.  It is reheated again at a constant pressure to state 4 before entering the second turbine and expands to 100% saturated vapor at state 5.  Finally, the heat is rejected to a condenser unit operating at a pressure of 0.01 MPa.  Both turbines operate at the same isentropic efficiency of 70% and the total work output from these two turbines is 1200 kJ/kg. (a) Determine the pressure at 3s, 3 and 4 (they are the same), (b) determine the enthalpy at 3, (c) Determine the enthalpy at 5, (d) determine the enthalpy at 4, (e) determine the temperature at 4, (f) determine the quality at 5s.

550C and 10 MPa

Boiler, 1

2

3

4

T

S

1

2

3

4

5, 100% saturated vapor

3s

5s

70% isentropic efficiency

P=0.01 MPa

5

70% isentropic efficiency

2. (25%) In a steam power plant, the steam exiting from a turbine has a 90% quality and it enters the condensing unit maintained at a constant pressure of 0.1 MPa.  Your mission as a design engineer is to design a tube-in-shell heat exchanger to condense completely this steam into saturated liquid (0% quality) at mass flow rate of 10 kg/s.  The following engineering specifications are provided for your reference: the steam is flowing inside the circular copper tube (k=400 W/m.K) with an inside diameter of 2 cm and an outside diameter of 2.4 cm.  The convection coefficients are 4000 W/m2.K and 2000 W/m2.K inside and the outside the pipe, respectively.  The cooling water enters the shell at a temperature of 20C and leaves at 40C.  (a) Write an equation of the heat flux transfer (evaluated at the outside surface A2) from the steam to the water.  (b) Determine the total tube length needed inside the heat exchanger. (c) Determine the mass flow rate of the cooling water if the specific heat for water is a constant at 4000 J/kg.K.  (d) What is the input power required for a pump to compressed the exiting saturated liquid from 0.1 MPa to 8 MPa?

20C

Condenser unit

40C

T,2

T,1

An 1000-W thin-film heater is sandwiched between two layers of insulation plates (A & B, kA=kB=10 W/m.K) as shown in the following.  We would like to maintain a constant surface temperature of 40C on the right side of the plate.  (a) Determine the surface temperature TA, (b) Determine the thickness of plate B if plate A is 50 cm thick, (c) Determine the temperature of the heater.  Assume 1-D heat transfer and steady state.

Heater, 1000W

h=20 W/m2.K

TO=20C

h=20 W/m2.K

TO=20C

A     B

TA=?

TB=40C

Surface area 1 m2

4. (25%) 1 kg of air, can be modeled as an ideal gas, undergoes a Carnot power cycle (1-2 isothermal compression, 2-3 isentropic compression, 3-4, isothermal expansion, and 4-1, isentropic expansion.)  At the beginning of the isothermal expansion, the temperature is 900 K and the pressure is 8 MPa.   The isothermal compression starts at a pressure of 0.5 MPa and a temperature of 450 K. (a) Sketch the T-S and P-v diagrams.  (b) Complete the following property table. (c) Determine the work developed for each process. (d) Determine the heat transfer at each process.  (e) Determine the thermal efficiency.  (R=0.287 kJ/kg.K, Cv=0.707 kJ/kg.K)

T

P

|  | P | T | V |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |

v

S

5. (25%) Two tanks (A & B) full of water have cross-sectional areas of 1m2 and 2 m2, respectively.  The water level of both tanks drop at the rate of 0.01 m/s the instant when someone suddenly opens the draining valves (both valves have the same opening area of 0.001 m2) at the bottom of both tanks as shown.  Neglect all losses, (a) Estimate the water level (hA, hB) for each tank at that instant. (b) How long will it take to empty both tanks? (Note: Assume both tanks A & B start at a water level of 5 m for parts (c) & (d)) (c) If tank C collects water from both tanks, what is the final temperature of tank C when both tanks A & B are empty. (d) What is the entropy generation of the process (neglect losses and heat transfer) (Cp=4 kJ/kg.K, =1000 kg/m3, water is an incompressible fluid)

AA=2 m2, TA=40C

AB=1 m2, TB=60C

hB

hA