EML 3015C Thermal Fluids I			Quiz 2			Fall 1999

A solar collector plate is receiving solar irradiation from the sun at a rate of G=500 W/m2 outside a space station.  It is known that this “opaque” collector has a reflectivity=0.2, what is its absorptivity =?  Assume its surface emissivity e=0.3, determine the steady state surface temperature of the collector plate when the temperature in the outer space can be approximated to be 0 K.  (Note: the collector has two surfaces, each side has a 1 m2 surface area).  Also, the outer space can be considered as a vacuum, therefore what should be the convective coefficient of the plate?  (q radiation = A (T4-T4surr), where =5.67x10-8 W/m2 K4.)

Girradiation=500 W/m2

α+ρ+τ=1, α=1−ρ=0.8 since ρ=0 (opaque surface)Energy balance:0=dEdt=qin−qout=αG−εσA(T4−Tsurr4)−hA(T−T∞)h=0 since the outside is a vacuum0.8(500)(1)=(0.3)(5.67×10−8)(2)(1)(T4−0)T=329(K)