EML 3015C Thermal-Fluid I 	Lab-10						fall 1999

Consider a Rankine power plant as shown.  For both turbines, solid line represents the actual processes of the cycle, while the dash line representing the idealized isentropic expansion processes.  (Note: States 3s and 5 fall on the saturated vapor line, Hint: Use constant pressure saturation table and find the closest values without interpolation.)  The steam is heated in the boiler at a constant pressure of 10 MPa to 550C and expands in the first turbine from state 2 to state 3.  It is reheated again at a constant pressure to state 4 before entering the second turbine and expands to 100% saturated vapor at state 5.  Finally, the heat is rejected to a condenser unit operating at a pressure of 0.01 MPa.  The first turbine operates at an isentropic efficiency of 73% and the combined actual work output from these two turbines is 1,230 kJ/kg. (a) Determine the pressure at 3s, 3 and 4 (Are they the same?), (b) determine the enthalpy at 3, (c) Determine the enthalpy at 5, (d) determine the enthalpy at 4, (e) determine the temperature at 4, (f) determine the quality at 5s, (g) the isentropic efficiency of the second turbine.

550C and 10 MPa

Boiler, 1

2

3

4

5

T

1

2

3

4

3s

5s

73% isentropic efficiency

isentropic efficiency ?

P=0.01 MPa

5

In the previous steam power plant, assume the steam exiting from a turbine has a 90% quality and it enters the condensing unit maintained at a constant pressure of 0.1 MPa.  Your mission as a design engineer is to design a tube-in-shell heat exchanger to condense completely this steam into saturated liquid (0% quality) at mass flow rate of 10 kg/s.  The following engineering specifications are provided for your reference: the steam is flowing inside the circular copper tube (k=400 W/m.K) with an inside diameter of 2 cm and an outside diameter of 2.4 cm.  The convection coefficients are 4,000 W/m2.K and 2,000 W/m2.K inside and the outside the pipe, respectively.  The cooling water enters the shell at a temperature of 20C and leaves at 30C.  (a) Write an equation of the heat flux transfer (evaluated at the outside surface A2) from the steam to the water.  (b) Determine the total tube length needed inside the heat exchanger. (c) Determine the mass flow rate of the cooling water if the specific heat for water is a constant at 4,000 J/kg.K.  (d) What is the input power required for a pump to compressed (isentropically) the exiting saturated liquid from 0.1 MPa back to 10 MPa?  Assume the temperature inside the condenser shell is at a constant temperature of 25 C, that the averaged water temperature between the inlet and the outlet.

20C

Condenser unit

30C

T,2

T,1