EML 3015C	Thermal Fluids I	Workshop Assignment 8		fall 1999

- Consider an ideal Rankine cycle using steam as the working fluid (see figure).  It is known that the pressure is 1 MPa, a constant, inside the boiler (2-3).  The steam leaves the boiler as 100% saturated vapor (state 3) when it enters the condenser.  Inside the condenser (4-1) the pressure is also a constant at 10 kPa.  (a) Determine the thermal efficiency of the cycle.  (b) Determine the amount of heat rejection in the condenser.

T

3

P2=P3=1 MPa

2

1

4

P1=P4=10 kPa

s

Qout

Qin

Boiler

Turbine

Compressor

(pump)

Heat exchanger

(Condenser)

1

2

3

4

Wout

Win