EML 3015C Thermal-Fluids I 	quiz-8					fall 1998

A steam turbine is operated at an isentropic efficiency of 74% as shown.  The steam exits from the boiler at a pressure of 0.6 MPa and a temperature of 300C.  After passing through the turbine, the steam enters the condenser that is maintained at a constant pressure of 0.1 MPa.  (a) Determine the actual work output of the turbine, (b) determine the actual temperature of the steam exiting from the turbine.

P3=0.6 MPa

T3=300C

ηs=Wactualh3−h4,S

Isentropic efficiency

Wout

P4=0.1 MPa

T4=?

P3=0.6 MPa, T3=300C, s3=7.3724 kJ/(kg K)=s4,S (through isentropic process), h3=3061.6 kJ/kg

P4=0.1 MPa, s4,S=7.3724 kJ/(kg K), T4100C, h4,S2676.2 kJ/kg

(a) Wactual=h3-h4= (h3-h4,S)=(0.74)(3061.6-2676.2)=285.2 kJ/kg

h4=h3-Wactual=2776.4 kJ/kg

at P4=0.1 MPa, T4=150C