EML 3015C Thermal Fluids I		Quiz 1			Fall 1999

A solid plate has a temperature profile (side view) as shown in the following figure, (a) what is the rate of heat transfer (in W) passing through the plate?  (Fourier's law of heat conduction: q= - kA(dT/dx), assume the cross-sectional area A=1 m2, k=100 W/m C)  (b) Which direction is the heat transfer?  Use Fourier's law to justify your answer.

T=100 C

T=50 C

x=0 m

x=1 m

x

Temp (C)

Air

TL=100 C

Air

TR=50 C

q=−kAdTdx=−(100)(1)50−1001−0=5,000(W)dTdx<0, −dTdx>0Flow in the positive x directionto the right

(C) If the air temperature on the left side of the block is suddenly decreased and the air temperature on the right side is increased, what will be the temperature distribution inside the block after a very short time period?  Sketch your answer in the following sketch.

Temp (C)

x

Original temperature profile

After the external temperatures change