EML 3015C Thermal-Fluids I 	quiz-9					fall 1998

A large metal plate is undergoing heat treatment.  As shown below, one side of the surface is maintained at a high temperature of 200 C while the other side is maintained at a low temperature of 0 C.  Assume the heat transfer is one-dimensional along the x direction and the treatment is a steady state process, determine (a) the rate of heat transfer through the plate, (b) the temperature at point A (0.2 m from the hot surface).  (k=100 W/m2 C)

Fourier's law: q=−kAdTdx, Steady heat diffusion equation (no generation): ∂2T∂x2+∂2T∂y2+∂2T∂z2=0

T(x=0)=200C

T(x=1 m)=0C

T(x=0.2m)=?

(a) q=−kAdTdx=−(100)(1)0−2001−0=20,000(W)=20(kW)(b) d2Tdx2=0, since T=T(x) onlyIntegrate twice:  T(x)=C1x+C2Use two boundary conditions: T(x=0)=200, T(x=1)=0C1=-200, C2=200T(x)=-200x+200=200(1-x)T(0.2)=200(1-0.2)=160(°C)