EML 3015C Thermal-fluids I	 Sample Test #3

Note: you have to show necessary steps leading to your answers to receive a full credit.

- (40%) Considering the ideal Rankine power plant as shown.  The steam mass flow rate is 60 kg/s.  The steam is first heated in the boiler at a constant pressure of 8 MPa and expands in the first turbine to become 100% saturated vapor before it is reheated again at a constant pressure of 0.8 MPa before entering the second turbine.  Finally, the heat is rejected to a condenser unit operating at a pressure of 10 kPa.   In order to remove the heat in the condenser unit, cooling water enters at 20C and leaves at 60C at a mass flow rate of 750 kg/s.  Determine (a) The power delivered by the pump, (b) The heat rejection of the condenser unit, (c) The work output of the first stage turbine, (d) the work output of the second stage turbine, (e) the total heat transfer into the power plant, (f) thermal efficiency of the power plant.

1st turbine

condenser

boiler

2

3

4

5

T

3

5

2nd turbine

4

2

pump

1

6

1

6

S

Tin=20C

Tout=60C

Cooling water

750 kg/s

(25%) Steam enters the turbine steadily at 8 MPa and 400C and leaves at a pressure of 1 MPa.  Determine (a) the work output if the process is isentropic.  If the actual turbine is operating at 70% isentropic efficiency, determine (b) the actual work output, (c) the actual exit temperature of the steam, (d) the entropy generation (assume adiabatic). Base your calculation on 1 kg/s of steady steam flow.

P=8 MPa

T=400C

turbine

P=1 MPa

3. (35%) A cylinder and piston assembly, containing 1 kg of air, is undergoing a complete cycle consisting of three reversible processes: 1-2 is an isentropc compression; 2-3 is an isothermal expansion from a pressure of 1 MPa to 100 kPa; and 3-1 is a constant pressure compression to a temperature of 50C.   Sketch the T-s and P-v diagrams representing this cycle.  Determine the total work and heat transfer of the cycle.  Assume ideal gas (Cp=1 kJ/kg K, Cv=0.72 kJ/kg K, R=0.287 kJ/kg K, k=Cp/Cv=1.4)

T

P

v

s

|  | P | T | v |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |

Formula Sheet

Entropy balance analysis: dsdt=QT+m1s1−m2s2+sgenerationEnergy equation: dEdt=dEgdt+Q+E1−E2−dWdt, E=m(h+V22+gz)Tds equation: Tds=du+Pdv=dh-vdPEntropy change of an idea gas:s2−s1=12cv(T)dTT+Rlnv2v1 or s2−s1=12cP(T)dTT−RlnP2P1Isentropic process of an ideal gas:T2T1=v1v2k−1, and T2T1=P2P1k−1kand P2P1=v1v2k, where k=cpcvFor a reversible process:Q=TdS,    W=PdV