EML 3015C Thermal-Fluids I Fall 1999	Sample Test #2B

- In a steam power plant, heat is continuously added to a boiler in order to supply 100% saturated vapor at 350C, 150 m/s to an adiabatic turbine.  The steam leaves the turbine at 20 kPa and 80% quality, and 20 m/s.  It is known that the turbine develops 10 MW power, determine (a) the mass flow rate of the steam, (b) the change in kinetic energy (per unit mass flow rate), (c) the turbine inlet and outlet areas.

350C

x1=100%

V1=150 m/s

20 kPa

x2=80%

V2= 20 m/s

dW/dt=10 MW

- Tank A (2 m3 in volume and is filled with 1 kg air at an initial temperature of 20C) is connected to tank B (1 m3 in volume and is totally evacuated) via a control valve.  Tank A is first compressed adiabatically by a piston to half its initial volume.  (a) Determine the temperature of tank A after the compression.  The control valve is then opened to let the air in tank A to expand adiabatically to fill both tanks A (still at half its original volume) and B.  (b) Determine the final pressure of the air after the system reaches thermal equilibrium.  (c) If 10 kJ of heat is added during the expansion process, what is the final temperature.

Tank A

Initial volume

2 m3

Tank B, 1 m3

- A piston-cylinder assembly initially contains 0.5 m3 of steam at 200C.  The piston is designed such that it maintains a constant pressure of 200 kPa (by moving the piston inward).  A valve is then opened and let steam to escape.  Heat is allowed to take place during the process to maintain at constant temperature of 200C.  The process continues until the final volume is 0.2 m3, determine (a) the amount of steam that has escaped and (b) the amount of heat transfer.