EML 3015C	Thermal Fluids I	Workshop Assignment 8		fall 1999

- Consider an ideal Rankine cycle using steam as the working fluid (see figure).  It is known that the pressure is 1 MPa, a constant, inside the boiler (2-3).  The steam leaves the boiler as 100% saturated vapor (state 3) when it enters the condenser.  Inside the condenser (4-1) the pressure is also a constant at 10 kPa.  (a) Determine the thermal efficiency of the cycle.  (b) Determine the amount of heat rejection in the condenser.

T

3

P2=P3=1 MPa

2

1

4

P1=P4=10 kPa

s

Qout

Qin

Boiler

Turbine

Compressor

(pump)

Heat exchanger

(Condenser)

1

2

3

4

Wout

Win

- From 1 to 2: The pump compressed isentropically the liquid to a higher pressure

Use the Tds equation: Tds=dh-vdP

Isentropic process: ds=0 and therefore dh=vdP

h2−h1=12dh=12vdP=v(P2−P1);  v is a constant since liquid water is incompressiblev=vf@saturation=0.00101(m3kg) from saturation table A-5

Work done by the pump Wpump=Win=h2-h1=v(p2-p1)=(0.00101)(1000-10)=1 kJ/kg

h1=hf @10kPa=191.8 kJ/kg

h2=h1+Wpump=192.8 kJ/kg

- From 2 to 3: Heat addition inside the boiler at a constant pressure

Qin=h3-h2,   h3=hg@1MPa =2778.1 kJ/kg

Qin=h3-h2=2778.1-192.8=2585.3 kJ/kg

- From 3 to 4: Doing work inside the turbine.  Assume adiabatic and irreversible, therefore, the process is isentropic.

s3=sg,3 @1MPa=6.5865 kJ/(kg K)

Inside the condenser, the fluid is a mixture of saturated liquid and saturated vapor

sf,4=0.6493 kJ/(kg K), sfg,4=7.5009 kJ/(kg K)

s4=sf,4+x4sfg,4, x4=s4−sf,4sfg,4=6.5865−0.64937.5009=0.79

hf,4=191.8 kJ/kg, hfg,4=2392.8 kJ/kg

h4=hf,4+x4hfg,4=191.8+(0.79)(2392.8)=2082.1 kJ/kg

Work output from the turbine: Wout=h3-h4=696 kJ/kg

- From 4 to 1: Inside the condenser, heat is rejected to condense the vapor into liquid water.

h1=hf,1=191.8 kJ/kg

Qout=h1=h4=191.8-2082.1=-1890.3(kJ/kg); heat loss to the cooling water

- The thermal efficiency is defined as the ratio between the net work and the heat addition:

ηth=WnetQin=Wout−WinQin=696−12585.3=0.269=26.9%