EML 3015C 	Thermal-Fluids I	Test 2		Name:________________________

Note: You have to show all necessary steps to receive full credit.

- A piston/cylinder assembly (with an initial volume of V1=0.1 m3, P1=1 MPa) contains 1 kg of a mixture of saturated liquid water and saturated steam as shown.  The system is undergoing through four processes 1-2-3-4-1 as indicated on the T-v diagram.  2-3 and 4-1 are constant volume process.  (40%)

  - Determine the initial quality (x1) and temperature (T1);

  - determine the temperature (T2) at the state 2;

  - determine the quality (x4) and temperature (T4) at the state 4;

  - determine the amount of the heat transfer Q12 added and work done W12 during the process 1-2;

  - determine the amount of the heat transfer Q23 lost and work done W23 during the process 2-3.

1

2

3

4

Const. Pressure line P=1 MPa

Const. Pressure line P=0.3 MPa

T

v

(a) v1=0.11=0.1 m3kg, P1=1 MPa, T1=179.9°CFrom saturated steam table: vf,1=0.001127, vg,1=0.1944v1=vf,1+x1(vg,1−vf,1), x=0.1−0.0011270.1944−0.001127=0.51(b) P3=0.3MPa, from saturation steam table: vg,3=0.6058=v2>vg,1Use superheated table and interpolate between 1,000°C & 1,100°C: P2=1MPa, v2=0.6058(m3kg)T2=1000+1100−10000.6335−0.5871(0.6058−0.5871)=1040.3(°C)(c) T4=133.55°C from saturation steam table with P4=0.3MPavf,4=0.001073, vg,4=0.6058,v4=v1=0.1m3kg (constant volume process)x4=0.1−0.0010730.6058−0.001073=0.165

(d) W12=12PdV=P1(V2−V1)=P1m(v2−v1)=(1000)(1)(0.6058−0.1)=505.8(kJ)ΔU12=U2−U1=m(u2−u1)=Q12−W12u1=uf,1+x1ufg,1=761.68+(0.51)(1822)=1690.9,   from saturated steam tableu2 needs to be interpolated from the superheated table between 1,000°C and 1,100°Cu2=4050.5+4255.1−4050.51100−1000(1040−1000)=4132.3(kJkg)Q12=(1)(4132.3−1690.9)+505.8=2947.2(kJ)(e) W23=23PdV=0ΔU23=m(u3−u2)=Q23Q23=m(ug,3−u2)=(1)(2543.6−4132.3)=−1588.7(kJ); ug,3 from saturated steam table with P=0.3 MPa

- A very large tank filled with superheated steam is used to charge a much smaller tank (volume of 1 m3) as shown.  It can be assumed that the thermal properties of the large tank do not change with time.  The small tank initially is filled with saturated vapor (100% quality, x1=1) at a pressure of 0.5 MPa.  In order to make sure the steam remains 100% saturated vapor (with x2=1), heat has to be removed from the small tank during this charging process. (30%)

  - Determine the final pressure (P2) when the charging process stops automatically;

  - determine the total mass (m2-m1) charged into the small tank during this period;

  - determine the total amount of the heat transfer removed, Q12.

  - determine the final temperature at the end of the charging process, T2.

Initially, V=1 m3

P1=0.5 MPa

100% saturated steam vapor

P=1.2 MPa

T=500C

superheated water vapor

(a) The charging process will stop when the inside pressure is equal to the outside pressure.P2=1.2MPa(b) From saturated steam table: vg,1=0.3749(m3kg),u1=ug,1=2561.2(kJkg)m1=Vvg,1=10.3749=2.667(kg)From saturated steam table: vg,2=0.1633(m3kg),u2=ug,2=2588.8(kJkg)m2=Vvg,2=10.1633=6.124(kg)mIN=m2−m1=6.124−2.667=3.457(kg)(c) E2−E1=m2u2−m1u1=mINhIN+Q12hIN=3476.3(kJkg) from superheated table with P=1.2 MPa & T=500° C.Q12=(6.124)(2588.8)−(2.667)(2561.2)−(3.457)(3476.3)=−2994.5(kJ)(d) The final temperature can be determined from the saturated steam table:P=1.2 MPa, Tsat=187.99°C

- The cylinder/piston assembly is filled with 1 kg of air (an ideal gas with R=0.287 kJ/kg K and Cv=0.72 kJ/kg, k=1.4.)  The initial pressure inside the cylinder is P1=100 kPa.  The system is going through three processes 1-2-3-1 to complete a cycle as shown.  (1-2:constant pressure and its volume expands from 1 m3 to 2 m3; 2-3: constant volume; 3-1: adiabatic.) (30%)

  - Determine the temperatures at all states (T1, T2 & T3);

  - determine the work (W12) and the heat transfer (Q12) between 1 and 2;

  - determine the work (W23) and the heat transfer (Q23) between 2 and 3;

  - determine the work (W31) between 3 and 1.

P

1

2

3

V

1 m3

2 m3

From 1-2: constant prerssure(b)W12=12PdV=P1(V2−V1)=(100)(2−1)=100(kJ)(a) T1=P1V1mR=(100)(1)(1)(0.287)=348(K)(a) T2=P2V2mR=(100)(2)(1)(0.287)=697(K)(b) ΔU12=U2−U1=mCV(T2−T1)=(1)(0.72)(697−348)=Q12−W12=Q12−100Q12=351.2(kJ)(c) W23=23PdV=0  No external workFrom 3 to 1, adiabatic process: T3T1=V1V3k−1,T3348=121.4−1=0.758(a) T3=263.7(K)(c) ΔU23=mCV(T3−T2)=Q23−W23Q23=(1)(0.72)(263.7−697)+0=−311.9(kJ)(d) Adiabatic process Q31=0ΔU31=Q31−W31W31=−ΔU31=mCV(T3−T1)=(1)(0.72)(263.7−348)=−60.7(kJ)

Energy balance equation:

Integrate the energy equation in time for a uniform flow process:

(E2−E1)+mouth+V22+gzout−minh+V22+gzin=Q12−W12

Mass conservation:

Ideal gas:

PV=mRT

H(enthalpy)=mCpT, U(energy)=mCvT

Adiabatic compression/expansion process for an ideal gas going from state A to state B:

TBTA=VAVBk−1=PBPA(k−1)k, PBPA=VAVBk

Saturation property calculation:

 = f + x (g -f) = f + x fg, where  is any thermodynamic property such as energy, enthalpy, specific volume, etc., and x is the quality.

Energy balance for a closed system:  Q-W=U

W=PdV