Flow past a flat plate has a laminar boundary layer.  The boundary layer growth can be described as , while the momentum thickness can be expressed as  where x is the distance measured from the leading edge of the plate, U is the free-stream velocity.  The boundary layer thickness is 1 cm at the end of the plate and the drag force D due to friction is 10 N.

- If the free stream velocity is doubled as U2U with all other parameters remain constant, determine the new boundary layer thickness at the end of the plate.

- Also determine the new drag force.

- If the free stream velocity is unchanged and the length of the plate is doubled as L2L, determine the new boundary layer thickness at the end of the plate

- Determine the new drag force.  (To make this problem easier, you can assume all parameters, such as velocity, viscosity, length & width of the plate, to be one (a constant) initially.)