Example:  Air enters a two-dimensional wind tunnel as shown.  Because of the presence of the boundary layer, fluid will be displaced away from the surface.  In order to maintain a constant velocity inside the tunnel, it is necessary to increase the cross-sectional size of the tunnel.

- Determine the channel height, H(x), as a function of the distance measured from the inlet of the tunnel, x.  The tunnel velocity is 10 m/s and the tunnel inlet height is 5 m. Assume the boundary layer has a profile u(y)=Usin(y

- What is the momentum thickness (x) of the boundary layer flow.

- If a sphere has a diameter of 0.1 m is placed in the center of the wind tunnel, what is the drag force exerted on the sphere.  air=1.2 kg/m3, =1.5x10-5 m2/s.

H(x)

Sphere, 0.1 m diameter

U=10 m/s

5 m

x

CD=24/Re, when Re<1

CD=24/Re0.646, when 1<Re<400

CD

Re=VD/

0.5

400

300,000