Example

Steel beams are used to support a bridge with a steady loading on each beam P=40,000 lbf.   Due to the relatively poor manufacturing quality, the cross-sectional of the available beams also vary with an average area A=1 in2and a standard deviation sA=0.05 in2.  It is known that the yield stress of the steel has a normal distribution with an average yield stress of σY=50 ksi=50,000 psi,and a standard deviation sσ=5 ksi,determine the probability for one of the beams to fail (that is, when the applied loading produces a stress higher than the yield stress of the steel).   Note: from Table 5.2 (chapter 5, p 228):sx1x2=x22sx12+x12sx22x22,sx−y=sx2+sy2

Solution:Define margin of safety Q as Q=σY−PA.  Therefore, Q=0 signifies the point of failurewhen the applied load equals to the yield stress of the support beamQ=σY−PA, Q=50,000−PA=50,000−40,0001=10,000sPA=P2sA2+A2sP212A2=(40,000)2(0.05)2+(1)2(0)212(1)24000000+0121=2,000(psi)sQ=sσ2+sPA2=(5000)2+(2000)2=5,385Rearrange Q into the standard z-variable: z=Q − QsQWhen Q≤0⇒failure occursz≤0−10,0005,385=−1.86⇒from z-normal distributionPr(z≤-1.86)=0.0314