EML 4550	Engineering Design Methods		Example		Fall 2007

Steel beams are used to support a bridge with fluctuating loading on each beam P such that the average load P=40,000 lbfand its standard deviation load sP=6,000 lbf.  Due to the relatively poor manufacturing quality, the cross-sectional of the available beams also vary with an average area A=1 in2and a standard deviation sA=0.08 in2.  It is known that the yield stress of the steel is σY=50 ksi=50,000 psi,determine the probability for one of the beams to fail (that is, when the applied loading produces a stress higher than the yield stress of the steel).

Define margin of safety Q as Q=σY−PA.  Therefore, Q=0 signifies the point of failurewhen the applied load equals to the yield stress of the support beamQ=σY−PA, Q=50,000−PA=50,000−40,0001=10,000sQ=sPA=P2sA2+A2sP2A2=(40,000)2(0.08)2+(1)2(6,000)212(1)210240000+36000000121=6,800(psi)Note: from Table 5.2 (chapter 5, p 228):sx1x2=x22sx12+x12sx22x22Rearrange Q into the standard z-variable: z=Q − QsQWhen Q≤0⇒failure occursz≤0−10,0006,800=−1.47⇒from z-normal distributionPr(z≤-1.47)=0.0708