EML 4421 Fundamentals of IC Engines and Propulsion Systems		Fall 2001

Homework 4	Due October 3

- Please download the Stanford Engine Simulation Program (ESP) from http://esp.stanford.edu/.  This is a simulation program for engine performance developed by Professor W. C. Reynolds at Stanford University.  Detailed information of the program can be found in chapter 8 of our textbook.  Please read and understand the basic function of the program.  The following assignments should be completed using Stanford ESP program:

- Working with ESP, using no manifold and any convenient configuration, and vary the point of ignition to 30, 25, 20, 15, 10, 5, and 0 CAD BTC, and plot the peak pressure and the CAD at which peak pressure occurs as a function of ignition CAD (on separate graphs).  Discuss implications for knock.  (This is essentially the problem 1.2 in the textbook.)

- Working with ESP, using no manifold and any convenient configuration, vary the inlet pressure from 0.37 atm (corresponding to idle) to a maximum of 1.5 atm (corresponding to the case of either supercharged or turbocharged).  Plot the peak pressure and volumetric efficiency (on separate graphs) as a function of the inlet pressure.  Discuss your results.  (This is essentially the problem 2.1 in the text)

- Show that the mass flow through a valve can be given by using isentropic relation and mass conservation:

- Show that, for a choked valve, the volumetric efficiency can be related to the inlet Mach index (Z) by:

, This expression is derived based on =1.4 and terminology defined in the lecture notes.