Summary

    - Physical Layer 

Media Types

Encoding/Decoding

Scrambling/Descrambling

    - Data Link Layer

Medium Access Control

Logic Link Control

Network Topologies

- Network Layer
Establishing/maintaining/Terminating network connection
routed protocols:	IP	IPX	Apple Talk
routing protocols:	RIP	RIPv2	OSPF	IGRP	EIGRP		IS-IS
 To find the best route

- Transport Layer
Establishing/maintaining/Terminating transport layer connection
TCP		SPX
segments and reassembles data 
flow control
error control

- Session Layer
Managing data exchange

Pause and resume communication

- Presentation Layer
Character code translation/ Data conversion

Data compression/expansion

Encryption/Decryption

- Application Layer

Provides services to application processes or users.

  - File Transfer Protocol (FTP)

  - Remote Login (Telnet)

  - Transaction Server

  - Network Management

  - WWW Server/VoD Server

  - Browser (HTTP)

  - Internet Telephone

Basic	Functions of Protocols

Encapsulation/Decapsulation

Segmentation/Reassembly

Connection Establishing

Flow Control

Error Control

Multiplexing

Synchronization

Sequencing

Addressing

### Troubleshooting

| Common problems |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Hardware |  |  | Software |  |  |  |  |
| Line open/short |  |  | Software incompatible |  |  |  |  |
| NIC setup error |  |  | Workstation virus |  |  |  |  |
| Mixed lines |  |  | Slugish speed |  |  |  |  |
| Crosstalk/noise |  |  | Duplicated IP |  |  |  |  |
| Bad contact |  |  | Mask setup error |  |  |  |  |
| Hub reverse |  |  | Protocol or driver error |  |  |  |  |
| Deteriorated terminator |  |  | Server not working |  |  |  |  |
| ###### Physical layer |  |  |  |  |  |  |  |
|  | line too short |  |  |  |  |  |  |
|  | line too long |  |  |  |  |  |  |
|  | terminator error |  |  |  |  |  |  |
|  | NIC out of order |  |  |  |  |  |  |
|  | crosstalk/noise |  |  |  |  |  |  |
|  | repeater error (check power?) |  |  |  |  |  |  |
| ###### Data link layer |  |  |  |  |  |  |  |
|  | bridge error |  |  |  |  |  |  |
|  | collisions too many |  |  |  |  |  |  |
|  | frame storm and broadcast storm |  |  |  |  |  |  |
|  | utilization too high |  |  |  | 40% |  |  |
|  | frame too short |  |  |  | NIC/AP incompatible |  |  |
|  | frame too long |  |  |  | improper ground |  |  |
|  | CRC error |  |  |  |  |  |  |
|  | frame headless (no preamble) |  |  |  | may cause CRC error as well |  |  |
| ###### Netwrok layer |  |  |  |  |  |  |  |
|  | Duplicated IP setup |  |  |  |  |  |  |
|  | Routing table error |  | (OSPF can improve) |  |  |  |  |
|  | Router down |  |  |  |  |  |  |
|  | Subnet mask setup error |  |  |  |  |  |  |
|  | ICMP (Internet Control Message Protocol) malfunction |  |  |  |  |  |  |
| ###### Application layer |  |  |  |  |  |  |  |
|  | Version incompatible |  |  |  |  |  |  |
|  | Virus |  |  |  |  |  |  |
|  | Poor software efficiency |  |  |  |  |  |  |
|  | Repeated R/W actions, queries |  |  |  |  |  |  |
|  | Cannot setup connection with another workstation |  |  |  |  |  |  |

Useful web sites

http://www.webdev.net/orca/nojava/faq.htm

http://www.arcetri.astro.it/irlab/doc/doc_pc/pc-hw-faq/eide.1

http://www.itheadline.com/

http://www.cs.wisc.edu/~ghost/aladdin/get601.html