Exercise 1	network=182.182.0.0	mask=255.255.255.0

Total subnetworks: 256

Available nodes in each network: 254

Exercise 2	network=182.182.0.0	mask=255.255.224.0

represent it in / format: /19

Total subnetworks: 8

Available nodes in each network: 2^13 - 2

Exercise 3	Network number=192.168.5.0 needs to have 5 PC’s in each subnetwork

1.	which class?	C

2.	maximum number of subnets= 32

3.	subnet mask = 255.255.255.248

Exercise 4	Write down the subnet mask in decimal numbers corresponding to / x

/9 ?	255.128.0.0			/18	 255.255.192.0		/25	255.255.255.128

/10	255.192.0.0			/19	 255.255.224.0		/26	255.255.255.192

/11	255.224.0.0			/20	 255.255.240.0		/27	255.255.255.224

/12	255.240.0.0			/21	 255.255.248.0		/28	255.255.255.240

/13	255.248.0.0			/22	 255.255.252.0		/29	255.255.255.248

/14	255.252.0.0			/23	 255.255.254.0		/30	255.255.255.252

/15	255.254.0.0			/24	 255.255.255.0		/31 ? (Not applicable)

Exercise 5	Write down the broadcast address of 10.1.1.1 with the following subnet masks.  Ignore the restriction of invalid first subnet and last subnet.

/8	10.255.255.255		/17	10.1.127.255			/25	10.1.1.127

/10	10.63.255.255			/18	10.1.63.255			/26	10.1.1.63

/11	10.31.255.255			/19	10.1.31.255			/27	10.1.1.31

/12	10.15.255.255			/20	10.1.15.255			/28	10.1.1.15

/13	10.7.255.255			/21	10.1.7.255			/29	10.1.1.7

/14	10.3.255.255			/22	10.1.3.255			/30	10.1.1.3

/15	10.1.255.255			/23	10.1.1.255

/16	10.1.255.255			/24	10.1.1.255

Exercise 6	Write down the broadcast of the following IP.  Which one of the following IP is not a valid IP?

128.1.2.3/29	128.1.2.7	16.65.30.1/20		16.65.31.255

128.5.6.7/28	128.5.6.15	16.65.60.1/19		16.65.63.255

128.4.5.6/30	128.4.5.7	16.65.100.1/18		16.65.127.255

128.3.4.5/26	128.3.4.63	16.65.200.1/17		16.65.255.255

128.10.15.20/27	128.10.15.31	16.65.1.1/16		16.65.255.255

128.5.160.3/23	128.5.161.255	16.65.0.0/15		16.65.255.255

128.6.7.10/25	128.6.7.127	16.65.3.4/14		16.67.255.255

128.7.6.4/22	128.7.7.255	16.65.100.200/13	16.71.255.255

128.6.27.8/21	128.6.31.255	16.65.128.255/12	16.79.255.255

Exercise 7	Which of the following IP does not belong to the same network as the others in the same group?

Group 1		Group2	Group3
	146.201.26.123/25	64.11.2.3/11	**64.10.2.3/12

**146.201.26.100/25	64.22.3.4/11	64.20.3.4/12

146.201.26.130/25	**64.33.4.5/11	64.30.4.5/12

Exercise 8	If a network needs to accommodate 2000 hosts what is the maximum slash number? What is the subnet mask in decimal number of this slash number?

2^10=1024   2^11=2048  32-11=21  ==> /21	255.255.248.0

Exercise 9	A PC has been assigned the IP 146.201.26.27 and subnet mask 255.255.255.0.  A packet with which of the following IP as its destination address would be accepted by this PC?

146.201.26.27

146.201.255.255

127.0.0.1

All of them

Ans: 146.201.26.27, 127.0.0.1, 146.201.26.255, 255.255.255.255

Exercise 10	What are the broadcast address of the following IP address?

- 10.10.10.10/30

10.10.10.11

- 10.100.100.100/25

10.100.100.127

- 10.80.0.0/12

10.95.255.255

- 10.30.30.1/15

10.31.255.255

- 10.10.10.10/21

10.10.15.255

- 10.10.10.10/19

10.10.31.255