Note: Default gateway or static routes should be properly set up on each PC, client or server.  (using ifconfig and route command on Linux or FreeBSD)

- FTP server 192.5.5.10
From 210.93.105.0 cannot access FTP server, allow all other traffics.

Ans:	On Lab_D:

access-list 101 deny tcp 210.93.105.0  0.0.0.255  192.5.5.10  0.0.0.0 eq 21
		access-list 101 permit ip any any
		interface s1

ip access-group 101 out
	or	interface e0
			ip access-group 101 in

Note: 210.93.105.0  0.0.0.255 can be replaced with any in this case.

- Web Server 210.93.105.10
From 205.7.5.0 cannot browse Web server, allow all other traffics.

Ans:	On Lab_A:

access-list 102 deny tcp 205.7.5.0  0.0.0.255  host  210.93.105.10 eq 80
		access-list 102 permit ip any any
		interface e1
			ip access-group 102 in

Note: 205.7.5.0  0.0.0.255 can be replaced with any in this case.

- Telnet Server 223.8.151.10
From 192.5.5.0 cannot telnet to 223.8.151.10, allow all other traffics.

Ans:	On Lab_A:

access-list 103 deny tcp 192.5.5.0  0.0.0.255  223.8.151.10  0.0.0.0 eq 23
		access-list 103 permit ip any any
		interface e0
			ip access-group 103 in

Note: 192.5.5.0  0.0.0.255 can be replaced with any in this case.

- FTP server 205.7.5.10
Allow only from 192.5.5.0 to access the FTP server, deny from all other sources to access the FTP server, allow all other traffics.

Ans:	On Lab_A:

access-list 104 permit tcp 192.5.5.0  0.0.0.255  host  205.7.5.10 eq 21
		access-list 104 deny tcp any  host  205.7.5.10 eq 21
		access-list 104 permit ip any any
		interface e1

ip access-group 104 out
OR:		access-list 141 permit tcp 192.5.5.0  0.0.0.255  host  205.7.5.10 eq 21
		access-list 141 permit ip any any
		access-list 142 deny tcp any  host  205.7.5.10 eq 21
		access-list 142 permit ip any any
		interface e0

ip access-group 141 in
		interface s0
			ip access-group 142 in
		Note: 192.5.5.0  0.0.0.255 can be replaced with any in this case.

- Web Server 219.17.100.10
Allow clients from the left (LAB A), deny clients from the right to access the web server.  Allow all other traffics.

Ans:	On Lab_C:

access-list 105 deny tcp any  host  219.17.100.10 eq 80
		access-list 105 permit ip any any
		interface s1

ip access-group 105 out
OR:		On Lab_B:
		access-list 105 deny tcp any  host  219.17.100.10 eq 80
		access-list 105 permit ip any any
		interface s0

ip access-group 105 in
	or	interface e0
			ip access-group 105 out

- FTP/Web/Telnet Server 223.8.151.10
(1) Allow all ip traffics except from the source of 205.7.5.0 ~ 205.7.5.127 to the server.
(2) Allow FTP ONLY from 192.5.5.0, deny all other FTP’s to the server.
(3) Allow telnet to the server from the right (210.93.105.0), deny from the left.
(4) Allow all sources to browse the web server except from 210.93.105.128~210.93.105.191
(5) Allow all other traffics.

Ans:	On Lab_B:

(1)	access-list 106 deny ip 205.7.5.0  0.0.0.127  host  223.8.151.10
	(2)	access-list 106 permit tcp 192.5.5.0  0.0.0.255  host  223.8.151.10 eq 21
	(2)	access-list 106 deny tcp any host  223.8.151.10 eq 21
	(3)	access-list 106 deny tcp any host  223.8.151.10 eq 23
	(5)	access-list 106 permit ip any any
		interface s0
			ip access-group 106 out
		On Lab_D:
	(2)	access-list 106 deny tcp any host  223.8.151.10 eq 21
	(4)	access-list 106 deny tcp 210.93.105.128 0.0.0.63 host  223.8.151.10 eq 80
	(5)	access-list 106 permit ip any any
		interface s1
			ip access-group 106 out
	or	interface e0
			ip access-group 106 in

Note: The clients in 223.8.151.0 cannot be stopped from accessing the server.

- FTP server at 192.5.5.12, web server at 205.7.5.12
Allow ONLY LAB A e0 to access the web server; allow ONLY LAB A e1 to access the FTP server; block all other FTP and Web traffics. 
Allow all ip traffics but block the traffic to 210.93.105.0.

Ans:	On Lab_A:

access-list 171 permit tcp 192.5.5.0  0.0.0.255  host  205.7.5.12 eq 80
		access-list 171 deny tcp any host  205.7.5.12 eq 80
		access-list 171 permit ip any any
		access-list 172 permit tcp 205.7.5.0  0.0.0.255  host  192.5.5.12 eq 21
		access-list 172 deny tcp any host  192.5.5.12 eq 21
		access-list 172 permit ip any any
		access-list 173 deny tcp any any eq 80
		access-list 173 deny tcp any any eq 21
		access-list 173 permit ip any any

interface e0
			ip access-group 171 in
	or	interface e1
			ip access-group 171 out
		interface e1
			ip access-group 172 in
	or	interface e0
			ip access-group 172 out
		interface s0
			ip access-group 173 in

Note: The two servers with different IP’s can be implemented on one network card if using linux.  For example: 	ifconfig eth0:1 192.5.5.12
							ifconfig eth0:2 205.7.5.12