Objective

‧Investigate the default configuration of a 2900 series switch.

Step 1 Enter privileged mode

Switch>enable

Switch#

Step 2 Examine the current switch configuration

Switch#show running-config

Switch#show interface VLAN 1

Step 3 Get IOS information

Switch#show version

Step 4 Examine the Fast Ethernet interfaces

#show interface fastethernet 0/26 (Note: this is a trunk port)

Or

#show interface ethernet 0/4 (Note: this is an access port)

Step 5 Examine VLAN information

Switch>show vlan

Step 8 Exit the switch

Switch#exit

Erasing and Reloading the Switch

1900 Series Switches

1. Remove VLAN Trunking Protocol (VTP) information.

#delete vtp

This command resets the switch with VTP parameters set to factory

defaults.

All other parameters will be unchanged.

Reset system with VTP parameters set to factory defaults, [Y]es or

[N]o?

Enter y and press Enter.

2. Remove the switch startup configuration from NVRAM.

#delete nvram

This command resets the switch with factory defaults. All system parameters will revert to their

default factory settings. All static and dynamic addresses will be removed.

Reset system with factory defaults, [Y]es or [N]o?

Enter y and press Enter.