Objective

‧Configure a switch with a name and an IP address.

‧Configure passwords to ensure that access to the CLI is secured.

‧Configure switch port speed and duplex properties for an interface.

‧Save the active configuration.

‧View the switch browser interface.

Step 1 Enter privileged mode

1900:

>enable

#

Step 3 Assign a name to the switch

Switch#configure terminal

Switch(config)#hostname ALSwitch

ALSwitch(config)#exit

Step 4 Examine the current running configuration

ALSwitch#show running-config

Step 6 Set the command mode passwords

1900:

ALSwitch(config)#enable password level 15 cisco

ALSwitch(config)#enable secret class

Step 7 Configure the layer 3 access to the switch

1900:

ALSwitch(config)#ip address 192.168.1.x 255.255.255.0

ALSwitch(config)#exit

b. Set the default gateway for the switch and the default management VLAN to 192.168.1.1 as

follows:

ALSwitch(config)#ip default-gateway 192.168.1.254

ALSwitch(config)#exit

Step 11 Exit the switch

ALSwitch#exit

(Optional) Do the following only before you do not continue committing switch configurations.

Erasing and Reloading the Switch

1900 Series Switches

1. Remove VLAN Trunking Protocol (VTP) information

#delete vtp

2. Remove the switch startup configuration from NVRAM.

#delete nvram