Objective

‧ Change passwords to the password recovery procedure be performed.

Catalyst 1900 and 2820

Note: This section does not apply to those Catalyst switches that do not have the Mode button in their front panel.

- To access the Systems Engineering menu, follow the procedure below:

  - Disconnect the power cord from the rear panel.

  - Press and hold the LED Mode button on the front panel.

  - While holding the LED Mode button, reconnect the power cord.

  - Release the LED Mode button one or two seconds after the LED above port 1x goes off. You will see the Diagnostic Console Logon screen as shown below:

-------------------------------------------------

Cisco Systems Diagnostic Console

Copyright(c) Cisco Systems, Inc. 1999

All rights reserved.

Ethernet Address: 00-E0-1E-7E-B4-40

-------------------------------------------------

Press Enter to continue.

  - Press Enter to display the Diagnostic Console - Systems Engineering menu.

You will see the following Systems Engineering menu:

Diagnostic Console - Systems Engineering

Operation firmware version: 8.00.00 Status: valid

Boot firmware version: 3.02

[C] Continue with standard system start up

[U] Upgrade operation firmware (XMODEM)

[S] System Debug Interface

Enter Selection:

  - The bold letters above show the Boot firmware version. In the example shown above, it is 3.02.

Firmware Version 1.10 and Later

Clearing the Password

- To clear your password, follow the steps below:

  - Power-cycle the switch.
After POST completes, the following prompt displays:
	Do you wish to clear the passwords? [Y]es or [N]o:

Note: You have ten seconds to respond. If you don't respond within that time, the Management Console Logon screen is automatically displayed.

  - Enter [Y]es to delete the existing password from Nonvolatile RAM (NVRAM).
Note: If you type [N]o, the existing password remains valid.

  - Assign a password from the switch management interfaces (management console or Command Line Interface (CLI)).

Viewing the Password

For firmware versions between 1.10 and 3.02, you can view the password you are trying to recover (instead of clearing it as described in the previous section).

- Access the Diagnostic Console - Systems Engineering menu using the same procedure shown Earlier Disconnect the power cord from the rear panel.

  - Press and hold the LED Mode button.

  - While holding the LED Mode button, reconnect the power cord.

  - Release the LED Mode button one or two seconds after the LED above port 1x goes off. You will see the following logon screen:

-------------------------------------------------

Cisco Systems Diagnostic Console

Copyright(c) Cisco Systems, Inc. 1999

All rights reserved.

Ethernet Address: 00-E0-1E-7E-B4-40

-------------------------------------------------

Press Enter to continue.

  - Press Enter to display the Diagnostic Console - Systems Engineering menu. You will see the following Systems Engineering menu:

Diagnostic Console - Systems Engineering

Operation firmware version: 8.00.00 Status: valid

Boot firmware version: 3.02

[C] Continue with standard system start up

[U] Upgrade operation firmware (XMODEM)

[S] System Debug Interface

Enter Selection:

  - Select [S] from the Diagnostic Console - Systems Engineering menu to display the System Debug Interface Menu. You can use this menu to display the factory-installed management console password and to reset the management console RS-232 interface or the entire switch to the factory defaults

Diagnostic Console - System Debug Interface

[G] Generic I/O

[M] Memory (CPU) I/O

[F] Return system to factory defaults

[R] Reset main console RS232 interface to 9600,8,1,N

[V] View Management Console password

[X] Exit to Previous Menu

Enter Selection:

  - Select [V] from the Diagnostic Console - System Debug Interface menu to display the management console password. The password will be displayed as shown below:

The current management console password is: CISCO

  - Now you can use the option [X] to return to the Diagnostic Console - Systems Engineering menu, and then use [C] to continue with the standard system startup which will take you to the User Interface menu. If you want to change the password, select the [M] (Modify password) option on the Console Settings menu. The system will ask you to enter the old password, and then the new password.