- wr er  or erase start-config then reload

- Define hostname and ip host list

- Assign all interfaces an IP according to the above topology.

- Define on each router to allow 5 telnet sessions with password “cisco”

- Apply RIP routing protocol, at each router 
show interface to check doubtable links
(check point 40% for the completion of the above steps)

- Apply IGRP only on FSU and FAMU routers with as number 100
(check point 10% for the completion of this step)

- Define default route on Tampa and Miami routers.
(check point 10% for the completion of this step)

- On UF, FSU and FAMU, define static routes as needed.
(check point 20% for the completion of this step, or when time is up)

- Assume an FTP server with ip 50.50.50.10 and assign ip 172.16.1.1/32 for Lo0 of Tampa. Add 172.16.0.0 to the rip routing.  Add ACL(s) so that the networks on the left side of FSU cannot access the ftp server and the pool of 34.34.34.0/15 cannot access Lo0 of Tampa.  Allow all other traffics. (20% for the completion of this step)

show ip route   sh run   ping   telnet   trace   sh int   sh ver