- wr er  or erase start-config then reload

- Define hostname and ip host list

- Assign all interfaces an IP according to the above topology.

- Define on each router to allow 5 telnet sessions with password “cisco”

- Apply RIP routing protocol, at each router 
show interface to check doubtable links
(check point 40% for the completion of the above steps)

- Apply IGRP on all routers with as number 100
(check point 10% for the completion of this step)

- Define static routes on UCF and USF to override the IGRP routes.
(check point 10% for the completion of this step)

- On UF, check the connectivity paths to USF; on UNF, check the path it takes to UCF.
(check point 10% for the completion of this step, or when time is up)

- Assume an FTP server with ip 34.34.0.10.  Add ACL(s) so that the networks on the left side of FSU, including 16.16.0.0/15, cannot access the ftp server and the pool of 34.34.0.0/15 cannot access Lo0 of UNF.  Allow all other traffics. (30% for the completion of this step)

show ip route   sh run   ping   telnet   trace   sh int   sh ver