- wr er  or  erase start-config  then  reload

- Define hostname and secret password as “class”

- Assign all interfaces an IP according to the above topology, shut down unused interfaces

- Assign ip host with the Loopback or Ethernet address

- Define on each router to allow 5 telnet sessions with password “cisco”

- Apply RIP routing protocol at each router and broadcast all connected networks
traceroute between UCF and FAMU, between UF and UNF, and between FSU and USF
(check point 40% for the completion of the above steps)

- Apply IGRP on all routers with as number 100 and include all networks except 70.70.70.1
traceroute between UCF and FAMU, between UF and UNF, and between FSU and USF
(check point 10% for the completion of this step)

- Define a static route to 70.70.70.1 on all routers, except UWF, to override the RIP route
(check point 10% for the completion of this step)

- Add ACL(s) so that the Ethernet of UCF cannot access the ftp server with ip 30.30.64.10
(check point 10% for the completion of this step)

- Add ACL(s) so that the Ethernet of USF cannot access all the web servers on 40.40.32.0/19
(check point 10% for the completion of this step)

- Add standard ACL(s) so that the link 67.67.128.0/20 only permits the traffic from 50.50.50.1 (check point 10% for the completion of this step)
Use extended traceroute command to trace from 50.50.50.1 to 70.70.70.1
Use extended traceroute command to trace from 60.60.60.1 to 70.70.70.1
(check point 10% for the completion of this step)

show ip route   sh run   ping   telnet   trace   sh int   sh ver